# Cartesian Robot — Overview

Introduction, Working Principle & Applications

## 1. Introduction

A Cartesian robot (also called a gantry robot or linear robot) operates on three linear axes (X, Y, Z) arranged at right angles. Known for their precision, rigidity, and simple kinematics, they excel in pick-and-place, CNC machining, and 3D printing applications.

## 2. Key Characteristics

| Parameter | Value |
| --- | --- |
| Robot Type | Cartesian (Linear Axes) |
| Degrees of Freedom | 3-DOF (X, Y, Z) + optional rotation |
| Motion Type | Linear along orthogonal axes |
| Workspace Shape | Rectangular / cubic volume |
| Payload Range | 0.5 kg – 500 kg |
| Repeatability | ±0.01 mm – ±0.05 mm |

## 3. Working Principle

The Cartesian robot moves its end effector by independently controlling three linear actuators. Each axis uses a ball screw or linear belt drive powered by a stepper or servo motor. Position feedback from linear encoders ensures accurate positioning. Motion coordination uses synchronized multi-axis interpolation.

## 4. Applications

- Pick-and-place operations in packaging and electronics

- 3D printing and additive manufacturing

- CNC plasma, laser, and waterjet cutting

- Gantry-type coordinate measuring machines (CMMs)

- Automated dispensing and adhesive application

- Laboratory automation and testing

## 5. Advantages & Limitations

| Aspect | Advantages | Limitations |
| --- | --- | --- |
| Workspace | Predictable rectangular | Limited to box-shaped workspace |
| Kinematics | Simple, easy to program | Large footprint for given workspace |
| Precision | Excellent positioning | Orientation limited without add-ons |
| Cost | Modular, scalable design | Multiple drive systems required |

Document maintained in the Robot Engineering Knowledge Base. All specifications are subject to revision.