# Motors

Servo, Stepper, BLDC Motors & Linear Actuators for Cartesian Axes

## 1. Motor Selection Overview

Cartesian robots use stepper motors for low-cost, open-loop positioning or servo motors where accuracy and speed are critical. BLDC motors with ball-screw drives suit high-duty-cycle, long-life applications.

## 2. Servo Motors

| Parameter | Details |
| --- | --- |
| Type | AC Servo (3-phase PMSM) |
| Voltage | 24 V DC – 400 V AC |
| Rated Torque | 0.1 Nm – 50 Nm |
| Max Speed | 1000 – 5000 RPM |
| Encoder Resolution | 17-bit or 23-bit absolute |
| Protection | IP54 / IP65 |
| Typical Brands | Fanuc, KUKA, Yaskawa, Mitsubishi, Panasonic |

## 3. Stepper Motors

Used where open-loop accuracy is sufficient. Common in smaller lab or DIY articulated/Cartesian robots.

| Parameter | NEMA 17 | NEMA 23 | NEMA 34 |
| --- | --- | --- | --- |
| Frame Size | 42 × 42 mm | 57 × 57 mm | 86 × 86 mm |
| Holding Torque | 0.3 – 0.6 Nm | 1.0 – 3.0 Nm | 4.0 – 12.0 Nm |
| Step Angle | 1.8° (200 steps) | 1.8° (200 steps) | 1.8° (200 steps) |
| Current/Phase | 0.8 – 2.0 A | 2.0 – 4.5 A | 4.0 – 8.0 A |
| Typical Use | Light Z-axis | Y carriage | X gantry |

## 4. BLDC Motors

Brushless DC motors provide high power density and long life. Ideal for high-speed wrist axes in articulated robots or high-duty-cycle Cartesian drives.

- Voltage range: 12 V – 48 V DC (low-voltage) or 100 V – 400 V AC (industrial)

- Speed range: 1000 – 20,000 RPM

- Integrated Hall sensors or sinusoidal encoder feedback

- Common controllers: ODrive, VESC, Maxon ESCON, Copley

## 5. Gearboxes

| Type | Gear Ratio | Backlash | Best Use |
| --- | --- | --- | --- |
| Harmonic Drive (Strain Wave) | 50:1 – 320:1 | < 1 arc-min | Rotary axis only |
| Planetary Gearbox | 3:1 – 100:1 | 1–5 arc-min | General purpose |
| Cycloidal Drive | 10:1 – 200:1 | < 1 arc-min | High-torque joints |
| Worm Gear | 10:1 – 100:1 | 5–30 arc-min | Self-locking axes |

All motor selections must be validated against peak torque requirements including inertia mismatch ratio ≤ 10:1.