# Motor Drivers

Servo Drivers, Stepper Drivers & Driver Datasheets

## 1. Servo Drivers

Servo drives (amplifiers) close the current, velocity, and position control loops. They receive commands via pulse-direction, analogue ±10 V, or fieldbus (EtherCAT, CANopen, PROFINET).

| Parameter | Typical Value |
| --- | --- |
| Input Voltage | Single-phase 200–240 V or 3-phase 380–480 V AC |
| Output Current | 1 A – 100 A (continuous) |
| Control Modes | Position / Velocity / Torque (CSP/CSV/CST) |
| Communication | EtherCAT, CANopen, PROFINET, Modbus |
| Regenerative Braking | Built-in resistor or external regen unit |
| Protection | OCP, OVP, OTP, STO (Safe Torque Off IEC 61800-5-2) |
| Popular Brands | Beckhoff AX5000, Siemens SINAMICS, Delta ASDA-B3, Panasonic MINAS |

## 2. Stepper Drivers

Stepper drivers supply phase currents to stepper motors. Microstepping reduces vibration and improves positional resolution up to 256× step division.

| Feature | Basic Driver | Microstepping Driver | Closed-Loop Driver |
| --- | --- | --- | --- |
| Step Resolution | Full / Half step | Up to 1/256 | Up to 1/256 |
| Current Control | Fixed | PWM chopper | Adaptive PWM |
| Encoder Feedback | None | None | Yes (stall detect) |
| Heat Dissipation | Passive | Passive/Active | Active |
| Examples | L298N, A4988 | DRV8825, TMC2208 | Leadshine EM806 |

## 3. BLDC / Brushless Motor Controllers

- ODrive v3.6: Open-source, dual-axis, 24–56 V, 60 A peak — ideal for hobby robots

- VESC 6 MkVI: High-performance, configurable, 8–60 V, 240 A peak

- Maxon ESCON 50/5: Precision servo controller for Maxon EC motors

- Copley Accelnet: EtherCAT fieldbus, STO certified, industrial grade

## 4. Driver Datasheet Index

| Driver Model | Manufacturer | Motor Type | Interface | Datasheet Location |
| --- | --- | --- | --- | --- |
| AX5206 | Beckhoff | AC Servo | EtherCAT | 03_Motor_Drivers/Driver_Datasheets/ |
| MINAS A6 | Panasonic | AC Servo | EtherCAT/CANopen | 03_Motor_Drivers/Driver_Datasheets/ |
| TMC5160 | Trinamic | Stepper | SPI | 03_Motor_Drivers/Driver_Datasheets/ |
| DM860H | Leadshine | Stepper | Pulse/Dir | 03_Motor_Drivers/Driver_Datasheets/ |
| ODrive v3.6 | ODrive Robotics | BLDC | USB/UART/CAN | 03_Motor_Drivers/Driver_Datasheets/ |

Always consult the latest manufacturer datasheet before wiring. Verify STO wiring for safety-rated applications per IEC 62061.