# Mechanical System

Linear Axes, Gantry Frame, End Effector, Rails & Carriages

## 1. Structural Overview

The Cartesian robot mechanical system comprises a rigid base/gantry frame, X-axis beam, Y-axis carriage, Z-axis column, and end-effector mounting plate. Linear motion is delivered via ball screws or belt drives on precision linear rails.

## 2. Major Sub-Assemblies

### 2.1 Gantry / Base Frame

Welded steel or anodised aluminium extrusion frame. Provides the primary X-axis travel rail mounting. Ground-level or elevated suspended configurations available.

### 2.2 X-Axis Linear Stage

Main horizontal axis. Ball-screw driven or rack-and-pinion for long travel. Typical travel range: 300 mm – 5000 mm. Supported by profile linear rail (25–45 mm rail width).

### 2.3 Y-Axis Carriage

Mounts on the X-axis. Drives the Z column horizontally. Uses dual-rail design for yaw stiffness. Belt drive common for high-speed applications.

### 2.4 Z-Axis Column

Vertical axis for end-effector height control. Telescopic or rigid column. Ball screw with anti-backlash nut standard. Travel range: 100 mm – 1000 mm typical.

### 2.5 End Effector Mounting

Standardised mounting plate (ISO/DIN flange). Optional 4th rotary axis (R-axis) for orientation control. Quick-change tooling compatible with pneumatic couplings.

## 3. Mechanical Specifications

| Component | Material | Key Parameter | Typical Value |
| --- | --- | --- | --- |
| Gantry Frame | Steel / Al Extrusion | Flatness | < 0.05 mm/m |
| Linear Rail | Hardened Steel | Width | 15 – 45 mm |
| Ball Screw | C3 grade steel | Lead | 5 – 20 mm |
| Linear Carriage | Steel block | Dynamic Load | 5 – 80 kN |
| Z-Column | Aluminium Profile | Travel | 100 – 1000 mm |
| End-Effector Plate | Aluminium / Steel | Flatness | < 0.02 mm |

Refer to CAD Section 10 for detailed drawings and STEP files of all mechanical components.