# Control System

STM32, Arduino, Raspberry Pi, PLC & Industrial Controllers

## 1. Control System Architecture

The robot control system implements a hierarchical structure: mission/task planner → trajectory generator → joint controller → motor driver → plant (mechanical system). Real-time closed-loop control is typically executed at 1 kHz – 8 kHz.

## 2. STM32 Microcontroller

| Feature | Details |
| --- | --- |
| Family | STM32F4 / STM32H7 / STM32G4 |
| Core | ARM Cortex-M4/M7, 168–480 MHz |
| Peripherals | TIM (PWM encoder), SPI, I²C, USART, CAN FD |
| Real-Time OS | FreeRTOS, ChibiOS, Zephyr RTOS |
| Use Case | Joint-level PID, encoder reading, PWM generation |
| Dev Tools | STM32CubeIDE, PlatformIO, Keil MDK |

## 3. Arduino Platform

- Arduino Mega 2560: 54 digital I/O, 16 analogue, suitable for small delta/SCARA/Cartesian

- Arduino Due: 32-bit ARM Cortex-M3, 84 MHz — improved for multi-axis control

- RAMPS 1.4 shield: 5× stepper drivers, 3× thermistor, 3× heater outputs — common in DIY robots

- Libraries: AccelStepper, Servo.h, Wire for I²C sensor integration

## 4. Raspberry Pi

Used as a high-level mission computer or vision processor, not typically for hard real-time joint control without a co-processor.

| Model | CPU | RAM | Best Use |
| --- | --- | --- | --- |
| RPi 4B | Cortex-A72 1.8 GHz | 2–8 GB | ROS2 + vision pipeline |
| RPi CM4 | Same as RPi 4 | 1–8 GB | Embedded integration |
| RPi 5 | Cortex-A76 2.4 GHz | 4–16 GB | High-throughput AI inference |

## 5. PLC & Industrial Controllers

- Siemens S7-1200/S7-1500: PROFINET/PROFIBUS, integrated safety, TIA Portal

- Allen-Bradley CompactLogix: EtherNet/IP, Studio 5000 programming

- Beckhoff CX series: EtherCAT master, TwinCAT 3 soft PLC — microsecond cycle times

- Omron NJ/NX series: EtherCAT, Sysmac Studio, motion control library built-in

- Industrial PC (IPC): PC-based control with real-time OS (Xenomai, PREEMPT-RT Linux)

Control system software is version-controlled in 12_Software/Firmware/. Hardware configurations are documented in 11_Electronics/Schematics/.