# Kinematics

Linear Kinematics, Workspace Envelope & Trajectory Planning

## 1. Kinematic Model

Cartesian robot kinematics is trivially defined: each axis is orthogonal and independent. The end-effector position (x, y, z) maps directly to the motor positions. No matrix inversion is required.

## 2. Axis Mapping

| Axis | Motor | Travel | Position Variable | Resolution |
| --- | --- | --- | --- | --- |
| X (horizontal) | Servo/Stepper | 500 – 5000 mm | x = steps × pitch | 0.01 mm |
| Y (cross) | Servo/Stepper | 300 – 2000 mm | y = steps × pitch | 0.01 mm |
| Z (vertical) | Servo/Stepper | 100 – 1000 mm | z = steps × pitch | 0.005 mm |
| R (optional rotation) | Servo | 360° | r = encoder counts | 0.01° |

## 3. Forward & Inverse Kinematics

For a Cartesian robot, FK and IK are identical — end-effector position equals the axis positions directly. p_EE = [x_motor, y_motor, z_motor]. No iterative solver is needed.

## 4. Workspace Envelope

The Cartesian workspace is a rectangular cuboid defined by axis travel limits. Dead zones around homing switches reduce usable travel by 5–10 mm per axis end.

## 5. Trajectory Planning

- Joint-space interpolation: polynomial (cubic / quintic / trapezoidal velocity profiles)

- Cartesian-space interpolation: linear (LINTERP) and circular (CIRINTERP) paths

- Via-point blending: corner rounding with defined blend radius

- Velocity/acceleration/jerk limits enforced per axis

- Coordinated multi-axis motion with electronic gearing

Kinematic solver implementations are in 08_Kinematics/ and simulation models are in 12_Software/Simulations/.