# Articulated Robot — Overview

Introduction, Working Principle & Applications

## 1. Introduction

An articulated robot is a robotic arm with rotary joints, typically configured as 6-DOF (degrees of freedom), enabling complex spatial movement. These robots are widely used in manufacturing, welding, painting, and material handling applications.

## 2. Key Characteristics

| Parameter | Value |
| --- | --- |
| Robot Type | Articulated (Rotary Joint) |
| Degrees of Freedom | 6-DOF (typical) |
| Motion Type | Rotational at each joint |
| Workspace Shape | Irregular spherical envelope |
| Payload Range | 0.5 kg – 2000 kg |
| Repeatability | ±0.02 mm – ±0.1 mm |

## 3. Working Principle

Each joint in the articulated robot is driven by a servo motor with a gearbox. Joint angle sensors (encoders) feed back position data to the controller, which computes joint torques through inverse dynamics. The controller executes motion profiles ensuring smooth end-effector trajectories.

## 4. Applications

- Spot welding and arc welding in automotive manufacturing

- Painting and surface coating

- Assembly of electronics and mechanical components

- Material handling, palletizing and de-palletizing

- Machine tending for CNC and injection moulding

- Surgical robotics and rehabilitation devices

## 5. Advantages & Limitations

| Aspect | Advantages | Limitations |
| --- | --- | --- |
| Workspace | Large, irregular envelope | Complex singularity regions |
| Flexibility | High — complex orientations | Harder to program |
| Speed | High for large payloads | Lower accuracy at high speed |
| Cost | Efficient per task | Higher initial investment |

Document maintained in the Robot Engineering Knowledge Base. All specifications are subject to revision.