# Mechanical System

Base Frame, Arms, End Effector, Joints, Bearings & Fasteners

## 1. Structural Overview

The articulated robot mechanical system consists of the base frame, shoulder, upper arm, elbow joint, lower arm (forearm), wrist, and end-effector flange. Each link is connected through rotary joints with precision bearings and driven by servo-gearbox assemblies.

## 2. Major Sub-Assemblies

### 2.1 Base Frame

Heavy cast iron or welded steel structure anchored to the floor. Provides vibration isolation and supports entire robot weight. Typical base diameter: 200 mm – 800 mm for 6-axis robots.

### 2.2 Upper Arms

Aluminium or carbon-fibre links connecting shoulder to elbow joints. Optimised for strength-to-weight ratio. Hollow sections route cables and pneumatic lines internally.

### 2.3 Lower Arms (Forearm)

Connects elbow to wrist. Typically parallelogram linkage design for improved rigidity. Houses wrist motor cables and feedback wiring.

### 2.4 End Effector

ISO 9283 standard flange (ISO 9283 / DIN ISO 9283). Tool changers, grippers, welding torches, or vacuum cups mount here. Maximum tool weight varies by rated payload.

### 2.5 Universal Joints & Wrist Assembly

3-axis wrist with roll-pitch-yaw motion. Uses crossed-roller bearings for high stiffness. Harmonic drives in wrist joints provide high gear reduction with zero backlash.

## 3. Mechanical Specifications

| Component | Material | Key Parameter | Typical Value |
| --- | --- | --- | --- |
| Base Frame | Cast Iron / Steel | Mass | 50 – 500 kg |
| Upper Arm Link | Aluminium Alloy 6061 | Length | 300 – 700 mm |
| Forearm Link | Carbon Fibre / Al | Length | 250 – 600 mm |
| End Effector Flange | Steel (ISO 9283) | Diameter | 40 – 200 mm |
| Wrist Bearings | Crossed-Roller | Radial Load | 500 – 5000 N |
| Joint Bearings | Angular Contact | C0 Rating | Per motor torque |

Refer to CAD Section 10 for detailed drawings and STEP files of all mechanical components.