# Motors

Servo, Stepper, BLDC Motors & Gearboxes for Articulated Joints

## 1. Motor Selection Overview

Articulated robots typically use AC servo motors on major axes (J1–J3) for high torque and servo or BLDC motors on minor wrist axes (J4–J6) for speed and compactness. Gearboxes (harmonic drive or planetary) are mandatory on all joints.

## 2. Servo Motors

| Parameter | Details |
| --- | --- |
| Type | AC Servo (3-phase PMSM) |
| Voltage | 200 V AC / 400 V AC |
| Rated Torque | 0.3 Nm – 200 Nm |
| Max Speed | 3000 – 6000 RPM |
| Encoder Resolution | 17-bit or 23-bit absolute |
| Protection | IP54 / IP65 |
| Typical Brands | Fanuc, KUKA, Yaskawa, Mitsubishi, Panasonic |

## 3. Stepper Motors

Used where open-loop accuracy is sufficient. Common in smaller lab or DIY articulated/Cartesian robots.

| Parameter | NEMA 17 | NEMA 23 | NEMA 34 |
| --- | --- | --- | --- |
| Frame Size | 42 × 42 mm | 57 × 57 mm | 86 × 86 mm |
| Holding Torque | 0.3 – 0.6 Nm | 1.0 – 3.0 Nm | 4.0 – 12.0 Nm |
| Step Angle | 1.8° (200 steps) | 1.8° (200 steps) | 1.8° (200 steps) |
| Current/Phase | 0.8 – 2.0 A | 2.0 – 4.5 A | 4.0 – 8.0 A |
| Typical Use | Wrist axes | Elbow axis | Shoulder (small) |

## 4. BLDC Motors

Brushless DC motors provide high power density and long life. Ideal for high-speed wrist axes in articulated robots or high-duty-cycle Cartesian drives.

- Voltage range: 12 V – 48 V DC (low-voltage) or 100 V – 400 V AC (industrial)

- Speed range: 1000 – 20,000 RPM

- Integrated Hall sensors or sinusoidal encoder feedback

- Common controllers: ODrive, VESC, Maxon ESCON, Copley

## 5. Gearboxes

| Type | Gear Ratio | Backlash | Best Use |
| --- | --- | --- | --- |
| Harmonic Drive (Strain Wave) | 50:1 – 320:1 | < 1 arc-min | All robot joints |
| Planetary Gearbox | 3:1 – 100:1 | 1–5 arc-min | General purpose |
| Cycloidal Drive | 10:1 – 200:1 | < 1 arc-min | High-torque joints |
| Worm Gear | 10:1 – 100:1 | 5–30 arc-min | Self-locking axes |

All motor selections must be validated against peak torque requirements including inertia mismatch ratio ≤ 10:1.