# Sensors

Encoders, Limit Switches, Proximity, Vision & Force Sensors

## 1. Encoders

Rotary encoders at each joint are the primary position feedback for the servo controller. Absolute multi-turn encoders are preferred to avoid homing on power-up.

| Parameter | Rotary Encoder |
| --- | --- |
| Type | Absolute Multi-Turn (magnetic or optical) |
| Resolution | 17-bit – 23-bit (131K – 8.4M PPR) |
| Interface | BiSS-C, EnDat 2.2, SSI, Hiperface |
| Supply Voltage | 5 V or 10–30 V DC |
| IP Rating | IP64 – IP67 |

## 2. Limit Switches

- Mechanical: NC/NO type, actuated by cam or dog — robust, low cost

- Optical (photoelectric): non-contact, fast response, immune to vibration

- Magnetic reed switch: contactless, suitable for hydraulic or dusty environments

- Mounting: axis home position and over-travel (positive / negative limits)

- Logic: typically wired into E-stop chain via safety relay (Category 3 / PLd)

## 3. Proximity Sensors

| Type | Sensing Distance | Target | Output | Typical Use |
| --- | --- | --- | --- | --- |
| Inductive | 1 – 40 mm | Metal only | PNP/NPN | Metal part detection |
| Capacitive | 1 – 20 mm | All materials | PNP/NPN | Non-metal / liquid |
| Magnetic (Hall) | 1 – 10 mm | Magnet | Digital | Pneumatic cylinder end |
| Ultrasonic | 20 – 8000 mm | Any | Analogue/Digital | Distance measurement |

## 4. Vision Systems

Machine vision enables part recognition, pose estimation, and quality inspection. Common camera placements:

- Eye-in-hand: camera mounted on the end effector — dynamic scene capture

- Eye-to-hand: fixed overhead camera — large field of view, stable calibration

- 3D depth cameras: Intel RealSense, Ensenso — for bin picking and depalletizing

- Barcode / QR readers: Cognex DataMan series for tracking and traceability

## 5. Force / Torque Sensors

6-axis force/torque sensors at the wrist enable compliant motion, assembly force control, and collision detection.

| Parameter | Specification |
| --- | --- |
| Measurement Axes | Fx, Fy, Fz, Mx, My, Mz |
| Force Range | 50 N – 6000 N (Fz) |
| Torque Range | 2 Nm – 200 Nm |
| Overload Protection | 300 – 1000% of rated load |
| Interface | Analogue ±10 V, EtherNet/IP, EtherCAT |
| Brands | ATI Industrial Automation, Robotiq FT 300, Schunk FTN |

Sensor calibration records must be stored in 16_Metadata/components.csv with date and certificate number.