# Power System

SMPS, Batteries, Voltage Regulators & Protection Circuits

## 1. Power Architecture

The robot power system supplies clean, regulated power to all subsystems: logic/control (5 V/3.3 V), I/O and sensors (12 V/24 V), motor drives (48 V DC or 230/400 V AC), and safety circuits. A centralised SMPS or distributed converters may be used.

## 2. Switched-Mode Power Supplies (SMPS)

| Output | Common Voltage | Typical Power | Use |
| --- | --- | --- | --- |
| Logic Supply | 5 V / 3.3 V | 5 – 20 W | Microcontrollers, sensors |
| I/O Supply | 24 V DC | 50 – 200 W | PLC I/O, solenoids, indicators |
| Low-Voltage Drive | 48 V DC | 100 – 500 W | BLDC, stepper motors |
| High-Voltage Drive | 230 / 400 V AC | 0.5 – 20 kW | AC servo drives |

## 3. Battery Backup & UPS

- LiPo / Li-ion 48 V packs: mobile robot applications, autonomous operation

- Lead-acid UPS: offline UPS for controller during mains interruption

- Supercapacitor module: energy buffer for regenerative braking peaks

- BMS (Battery Management System): cell balancing, overcharge/overdischarge protection

## 4. Voltage Regulators

| Type | Application |
| --- | --- |
| LDO (e.g., AMS1117) | Low-dropout for 3.3 V / 5 V logic on PCBs |
| Buck Converter (e.g., LM2596) | 24 V → 5 V, efficient step-down |
| Boost Converter | Battery 12 V → 24 V for I/O rail |
| Isolated DC-DC (e.g., Murata) | CAN bus isolated 5 V rail |
| DIN-Rail PSU (e.g., PULS, Phoenix) | Industrial 24 V / 48 V distribution |

## 5. Protection Circuits

- Fuses / Circuit Breakers: per-axis fusing at motor drive inputs (type B/C MCB)

- TVS Diodes: transient voltage suppression on all signal and power lines

- Inrush Current Limiter (NTC thermistor or active circuit): prevents SMPS damage at power-on

- Emergency Stop Circuit: dual-channel E-stop with safety relay (e.g., Pilz PNOZ, Schmersal) — Category 3 / PLd minimum

- Regenerative Dump Resistor: dissipates braking energy from servo drives

- EMI Filter: IEC inlet filter on mains supply to meet CE/FCC radiated emissions

Power system schematics are in 11_Electronics/Schematics/. BOM with part numbers in 11_Electronics/BOM/.