# Communication

CAN, RS485, Ethernet & Wireless Protocols

## 1. Communication Stack Overview

The robot communication system spans multiple layers: field-level (sensor/actuator bus), control-level (inter-controller), and supervisory-level (HMI / SCADA / cloud). Protocol selection depends on cycle time, cable distance, and legacy compatibility.

## 2. CAN / CAN FD

| Parameter | CAN 2.0B |
| --- | --- |
| Max Bit Rate | 1 Mbit/s |
| Max Nodes | 110 |
| Max Cable Length | 40 m @ 1 Mbit/s |
| Frame ID | 11-bit / 29-bit |
| Use Case | CANopen motion control |
| Termination | 120 Ω at both ends |

## 3. RS-485 / Modbus RTU

- Half-duplex differential pair — immune to common-mode noise

- Distance: up to 1200 m at 100 kbit/s

- Up to 32 (standard) or 256 (with repeaters) nodes on one bus

- Modbus RTU framing: common for motor drives, PLCs, HMIs

- UART settings: 9600 – 115200 baud, 8N1 typically

## 4. Industrial Ethernet Protocols

| Protocol | Cycle Time | Topology | Use |
| --- | --- | --- | --- |
| EtherCAT | < 100 µs | Line / Tree / Ring | Servo synchronisation |
| PROFINET RT | 1 – 10 ms | Star / Ring | Siemens PLC systems |
| EtherNet/IP | 1 – 10 ms | Star | Allen-Bradley systems |
| Modbus TCP | 10 – 100 ms | Star | Supervisory / HMI |
| OPC UA | Variable | Client-Server | Cloud / Industry 4.0 |

## 5. Wireless Communication

- Wi-Fi 802.11ac/ax (5 GHz): robot web interface, firmware OTA, non-real-time data

- Bluetooth 5.0 / BLE: handheld teach pendant, mobile app pairing

- Zigbee / Thread: low-power sensor nodes (I/O expansion, tool sensors)

- 5G / LTE: cloud connectivity for telemetry, remote diagnostics, fleet management

- WirelessHART: industrial wireless sensor integration

All fieldbus wiring diagrams are in 11_Electronics/Wiring_Diagrams/. Network topology maps in 07_Communication/ sub-folders.