# Kinematics

Forward & Inverse Kinematics, Workspace & Trajectory Planning

## 1. Kinematic Model

Articulated robot kinematics is described using the Denavit-Hartenberg (DH) convention. The DH parameter table defines joint angles (θ), link lengths (a), link offsets (d), and twist angles (α) for each of the 6 joints.

## 2. DH Parameters (6-DOF Example)

| Joint | a (mm) | α (°) | d (mm) | θ (variable) |
| --- | --- | --- | --- | --- |
| J1 (Base Rotation) | 0 | 90 | d1 | θ1 |
| J2 (Shoulder) | a2 | 0 | 0 | θ2 |
| J3 (Elbow) | a3 | 0 | 0 | θ3 |
| J4 (Wrist Roll) | 0 | −90 | d4 | θ4 |
| J5 (Wrist Pitch) | 0 | 90 | 0 | θ5 |
| J6 (Tool Flange) | 0 | 0 | d6 | θ6 |

## 3. Forward Kinematics

Forward kinematics computes end-effector pose T₀₆ from joint angles [θ1…θ6] using the product of homogeneous transformation matrices: T₀₆ = T₀¹ · T¹² · T²³ · T³⁴ · T⁴⁵ · T⁵⁶.

## 4. Inverse Kinematics

IK computes joint angles from a desired end-effector pose. Analytical solutions exist for standard configurations (spherical wrist). Numerical solvers (Jacobian pseudoinverse, CCD) handle general cases. Singularity handling is critical near wrist singularities.

## 5. Trajectory Planning

- Joint-space interpolation: polynomial (cubic / quintic / trapezoidal velocity profiles)

- Cartesian-space interpolation: linear (LINTERP) and circular (CIRINTERP) paths

- Via-point blending: corner rounding with defined blend radius

- Velocity/acceleration/jerk limits enforced per axis

- Singularity avoidance using damped least-squares Jacobian

Kinematic solver implementations are in 08_Kinematics/ and simulation models are in 12_Software/Simulations/.