# Electronics

Schematics, PCB Designs, BOM & Wiring Diagrams

## 1. Schematics

All electrical schematics follow IEC 60617 symbols. Schematic capture is performed in KiCad 7 or Altium Designer. Multi-sheet schematics are organised by functional block.

| Sheet | Content |
| --- | --- |
| Sheet 1 — Power Distribution | Mains input, EMI filter, SMPS, fusing, E-stop relay |
| Sheet 2 — Motor Drives | Servo/stepper driver connections, DC bus, regen resistor |
| Sheet 3 — Control Board | MCU, CAN transceiver, RS-485, SPI encoder interface |
| Sheet 4 — Sensor I/O | Encoders, limit switches, proximity, force sensor wiring |
| Sheet 5 — Communication | Ethernet, CAN, RS-485 bus topology |
| Sheet 6 — Safety | E-stop chain, STO wiring, safety relay, door interlock |

## 2. PCB Designs

- Controller board: STM32H7 + CAN FD transceiver + RS-485 + encoder headers — 4-layer, 100 × 80 mm

- Power distribution board: 24 V fused bus + 5 V / 3.3 V regulators + connector strip — 2-layer

- I/O expansion board: 16 DI / 8 DO opto-isolated, screw terminals — DIN-rail mount

- Sensor signal conditioning: amplifier for force sensor bridge, anti-aliasing filter — 4-layer

## 3. Bill of Materials (BOM)

| Ref | Description | Manufacturer | Part Number | Qty |
| --- | --- | --- | --- | --- |
| U1 | STM32H743VIT6 MCU | STMicroelectronics | STM32H743VIT6 | 1 |
| U2 | CAN FD Transceiver | TI | TCAN1044VDRQ1 | 2 |
| U3 | RS-485 Transceiver | Maxim | MAX3485ESA | 2 |
| L1–L6 | 24 V Power LED Indicator | Lumex | SSL-LX5093ID | 6 |
| F1–F6 | 5 A PCB Fuse | Bel Fuse | 0685P5000-01 | 6 |

## 4. Wiring Diagrams

- Point-to-point wiring: each wire labelled with source terminal, destination terminal, gauge, and colour per IEC 60445

- Cable types: shielded twisted pair for encoders (STP 0.25 mm²), power cable 1.5–6 mm² for motor phases

- Drag chain sizing: minimum 10% free space in cable carrier for thermal expansion

- Cable numbering: prefix P = power, S = signal, F = fieldbus, following IEC 81346-1

PCB Gerber files, BOM exports, and schematic PDFs are released under revision control in 11_Electronics/.