# Manufacturing

CNC, 3D Printing, Laser Cutting & Assembly Guides

## 1. CNC Machining

Structural parts requiring tight tolerances (joint housings, bearing seats, ball-screw mounts) are CNC machined from aluminium alloy 6061-T6, 7075-T6, or mild steel.

| Parameter | Specification |
| --- | --- |
| Positional Tolerance | IT7 (±0.018 mm) on bearing seats |
| Surface Roughness | Ra 0.8 µm on mating faces, Ra 1.6 µm general |
| Thread Specification | M4 – M16, ISO 965 tolerance class 6H |
| Chamfers | 0.5 × 45° standard, 1 × 45° on assembly edges |
| Datum Strategy | 3-2-1 datum referencing on all machining drawings |

## 2. 3D Printing

- FDM (Fused Deposition Modelling): PLA+ for mockups; PETG/ABS for functional brackets

- SLS (Selective Laser Sintering): Nylon PA12 for complex cable management clips

- SLA/MSLA (Resin): high-detail parts, encoder mounts, sensor housings

- Layer height: 0.2 mm FDM standard; 0.05 mm resin for precision

- Infill: 30% gyroid for general; 80% rectilinear for load-bearing parts

## 3. Laser Cutting

Sheet metal panels, mounting plates, and cable trays are laser cut from 1.5–6 mm mild steel or 2–4 mm aluminium sheet.

| Material | Thickness | Kerf Width | Post-Process |
| --- | --- | --- | --- |
| Mild Steel S235 | 2 – 6 mm | 0.2 – 0.4 mm | Powder coat / Zinc plate |
| Aluminium 5052 | 2 – 4 mm | 0.15 – 0.3 mm | Anodise or clear lacquer |
| Stainless 304 | 1.5 – 3 mm | 0.2 – 0.35 mm | Passivation |
| Acrylic (PMMA) | 3 – 6 mm | 0.1 – 0.2 mm | Flame polished edges |

## 4. Assembly Guides

- Unpack and inspect all components against BOM (11_Electronics/BOM/)

- Assemble base frame and verify flatness with precision level (< 0.05 mm/m)

- Install linear rails / joint bearings — clean mounting faces with IPA

- Mount motors and align couplings — check runout < 0.05 mm TIR

- Route cables through drag chains; label with IEC 60445 numbering scheme

- Install encoders, sensors, and limit switches; verify functionality on bench

- Perform geometric calibration (refer to Section 09_Control_Algorithms/Calibration)

- Conduct dry run at 10% speed; check for collisions and interference

- Full speed and payload acceptance test; record results in test report

Manufacturing drawings with GD&T callouts are in 10_CAD/. Assembly torque specifications use ISO 4762 values unless otherwise noted.