COMPONENT ARCHITECTURE DOCUMENT

AGV-X200

Automatic Guided Vehicle (AGV-X200)

Version 1.0  |  2025

Industrial Robotics Division

# 1. System Overview

This document provides an architectural breakdown of the major subsystems and components of the Automatic Guided Vehicle (AGV-X200). Each subsystem is described in terms of its function, interfaces, and integration dependencies.

# 2. Mechanical Subsystem

## 2.1 Chassis & Frame

- Structural frame: Welded 6061-T6 aluminium alloy with steel reinforcement plates at load points

- Top deck: Flat 12 mm aluminium plate with M12 grid mounting pattern (100 mm pitch) for payload attachment

- Modular front/rear bumper sections with shock-absorbing foam insert

- IP54 sealed enclosure for electronics bay; positive-pressure ventilation with HEPA filter

## 2.2 Drive Assembly

- Drive configuration: 2× differential hub motors (200 W each) or 4× Mecanum wheels (150 W each)

- Hub motor: BLDC, 48V, 200W, with integrated hall-effect encoder (4096 PPR)

- Mecanum wheel: 200 mm diameter polyurethane rollers, aluminium hub, 30° roller angle

- Caster wheels: 2× fixed and 2× swivel nylon casters for chassis stability

## 2.3 Navigation Deck

- Primary LiDAR: 2D scanning LiDAR, 360° FOV, 30 m range, 10 Hz scan rate, USB/Ethernet interface

- Secondary sensors: 4× ultrasonic sensors (front/rear) for close-range (<500 mm) obstacle detection

- Safety bumper: Resistive contact strip, 270° coverage, triggers E-Stop on contact

- Optional 3D LiDAR upgrade: 16-channel, 100 m range for multi-level environment mapping

# 3. Power Subsystem

- Battery pack: 48V / 100Ah LiFePO4 in slide-out hot-swap cartridge

- BMS: Integrated Battery Management System with cell balancing, SOC estimation, thermal monitoring

- DC-DC converters: 48V → 24V (10A, logic/sensors), 48V → 12V (5A, communication), 48V → 5V (3A, USB)

- Charging port: Anderson SB120 connector, compatible with auto-docking floor charger

- Power distribution board: Fused per subsystem with individual enable relays

# 4. Compute & Control Subsystem

- Main compute: Industrial SBC (x86, quad-core 2.4 GHz, 8 GB RAM, 64 GB eMMC) running Ubuntu 22.04 + ROS2 Humble

- Safety PLC: Pilz PNOZ-m B0 safety relay module; certified SIL 2 / PLd

- Motor controllers: 2× FOC brushless motor driver, CAN bus interface, 100 Hz control loop

- Navigation stack: ROS2 Nav2, SLAM Toolbox, AMCL localisation

- Mission management: VDA 5050-compliant order manager with MQTT broker

# 5. Communication Subsystem

- Primary: Wi-Fi 6 (802.11ax) dual-band (2.4/5 GHz) with WPA3 enterprise auth

- Secondary: Gigabit Ethernet (RJ45, isolated) for dock-side data transfer

- Internal bus: CAN 2.0B (1 Mbps) for motor/sensor integration; I2C for peripheral expansion

- Protocols: MQTT 3.1.1, ROS2 DDS (FastDDS), REST API (JSON over HTTPS)

# 6. Safety Subsystem

- Hardware E-Stop: Dual-channel, hardwired, 40 ms response; requires manual reset

- Safety laser scanner: 270° protective field; configurable zone sets via safety PLC

- Software watchdog: Heartbeat monitor with 500 ms timeout → controlled deceleration → stop

- Anti-collision: Dynamic obstacle inflation in costmap; minimum 200 mm clearance enforced