BILL OF COMPONENTS (BOC)

AGV-X200

Automatic Guided Vehicle (AGV-X200)

Version 1.0  |  2025

Industrial Robotics Division

# Component Register

The following table lists all major components by subsystem. Part numbers, supplier model references, and key parameters are provided for procurement and integration reference.

| Part ID | Name | Model/Supplier | Subsystem | Quantity | Key Parameters |
| --- | --- | --- | --- | --- | --- |
| NAV-01 | LiDAR Sensor (Primary) | SLAMTEC RPLIDAR A3M1 | Navigation | 1 | 360° 2D, 25m, 10Hz |
| NAV-02 | Safety Laser Scanner | SICK S300 Expert | Safety | 1 | 270°, 7m protective |
| NAV-03 | Ultrasonic Sensor Array | MaxBotix MB7076 ×4 | Navigation | 1 | 2cm–7m, IP67 |
| NAV-04 | IMU | VectorNav VN-100 | Navigation | 1 | 6-DOF, 200Hz, CAN |
| DRV-01 | Hub Motor (Differential) | T-Motor U8 48V 200W ×2 | Drive | 1 | BLDC, 4096 PPR encoder |
| DRV-02 | Mecanum Wheel Set (optional) | Rotacaster 200mm ×4 | Drive | 1 | PU roller, 150W motor |
| DRV-03 | Motor Driver | ODrive Pro ×2 | Drive | 1 | FOC, 48V, 60A, CAN |
| PWR-01 | Battery Pack | CATL LiFePO4 48V/100Ah | Power | 1 | Hot-swap, BMS integrated |
| PWR-02 | DC-DC Converter 48→24V | Vicor DCM 48V 240W | Power | 1 | 95% efficiency, 10A |
| PWR-03 | DC-DC Converter 48→12V | Mean Well SD-100D-12 | Power | 1 | 100W, 8.5A |
| PWR-04 | Floor Charger | Wiferion etaLINK 3000 | Power | 1 | 3kW inductive auto-dock |
| CPU-01 | Main Compute SBC | Advantech MIC-770V3 | Compute | 1 | i7-1185G7, 8GB, 64GB eMMC |
| CPU-02 | Safety PLC | Pilz PNOZ-m B0 | Safety | 1 | SIL2/PLd, 4 OSSD |
| CMM-01 | Wi-Fi 6 Module | Intel AX210 mini-PCIe | Comms | 1 | 802.11ax, BT5.2 |
| CMM-02 | CAN Transceiver | PEAK PCAN-USB Pro | Comms | 1 | CAN 2.0B, 1Mbps |
| SEN-01 | Bumper Contact Strip | ASO SAFEYE Contact | Safety | 1 | 270°, 24V OSSD |
| ACT-01 | Signal Tower | Werma 802 LED | Indication | 1 | 3-color, 24V, buzzer |
| MECH-01 | Chassis Frame | Custom 6061-T6 Al/Steel | Mechanical | 1 | 500kg load rated |

# Procurement Notes

- All electronic components should be sourced from authorised distributors to ensure authenticity and warranty coverage.

- Safety-certified components (PLd/SIL2 or higher) must not be substituted without re-certification of the affected safety function.

- Minimum shelf life for LiFePO4 battery packs is 6 months from manufacture date at delivery; verify before acceptance.

- LiDAR sensors are calibrated at the factory; field replacement requires recalibration against the map reference frame.