LIMITATIONS & CONSTRAINTS DOCUMENT

AGV-X200

Automatic Guided Vehicle (AGV-X200)

Version 1.0  |  2025

Industrial Robotics Division

# 1. Introduction

This document describes the operational constraints, environmental restrictions, and technical limitations of the Automatic Guided Vehicle (AGV-X200). Understanding these limitations is essential for safe deployment, system integration, and expectation management during commissioning.

# 2. Operational Limitations

## 2.1 Navigation & Environment

- LiDAR SLAM requires a minimum of 3 distinct static landmarks per 10 m² for reliable localisation; featureless corridors (e.g., blank white walls) may cause drift exceeding ±50 mm.

- Map integrity degrades in dynamic environments with >20% temporary obstacle occupancy; map re-acquisition is required after significant layout changes.

- QR marker fallback requires markers to be clean, undamaged, and illuminated to at least 300 lux; soiled or reflective surfaces cause read failures.

- Magnetic tape guidance (optional kit) loses effectiveness above 5 mm ramp angle deviation between tape and sensor.

## 2.2 Speed & Payload

- Maximum speed is electronically limited to 0.8 m/s within 1.5 m of detected personnel per ISO 3691-4 proximity rules.

- Payload capacity of 500 kg is valid only on flat surfaces (gradient ≤ 2°); graded floors reduce effective payload by ~15% per degree of incline.

- Rapid acceleration (>0.3 m/s²) with unstable top-mounted loads (e.g., liquid containers) is not recommended without spill containment measures.

## 2.3 Battery & Power

- Operating at ambient temperatures below 5°C reduces effective battery capacity by up to 30%; below 0°C operation is not supported.

- Fast charging (2 hours) reduces battery cycle life to ~800 cycles vs ~2000 cycles at standard charge rate.

- Wireless charging pad variant has a positional tolerance of ±20 mm for coupling; mis-docking by more than ±30 mm results in no-charge event.

## 2.4 Fleet Management

- Maximum certified fleet size per single broker instance is 32 vehicles; larger fleets require a clustered broker with added latency.

- Wi-Fi coverage gaps exceeding 500 ms cause the vehicle to enter safe-stop mode; 5 GHz band is mandatory for latency-sensitive tasks.

- Cross-level operations (multi-floor) require dedicated elevator integration module; not included in base SKU.

# 3. Environmental Restrictions

| Parameter | Supported Range | Beyond Range Impact |
| --- | --- | --- |
| Temperature (operating) | -10°C to +50°C | Battery failure, sensor drift |
| Temperature (storage) | -20°C to +60°C | Seal degradation |
| Humidity (non-condensing) | 10% to 90% RH | Electronics corrosion risk |
| Floor flatness | ≤ ±5 mm per 2 m | Localisation error, tipping risk |
| Floor load rating | ≥ 1200 kg/m² | Floor damage, instability |
| Ambient light (LiDAR) | Any (LiDAR active) | Reduced for QR fallback below 200 lux |
| EMI environment | Industrial Zone A | Wi-Fi interference, sensor noise above Zone B |
| Altitude | ≤ 2000 m ASL | Battery cooling derating required above |

# 4. Integration Limitations

- The native fleet management API supports only MQTT 3.1.1; MQTT 5.0 and AMQP require a protocol bridge (not supplied).

- Third-party ERP systems must implement the AGV-IF interface standard (VDA 5050) for certified order management; custom REST integrations are unsupported by warranty.

- A maximum of 16 parallel mission queues are supported per vehicle; exceeding this causes oldest missions to be silently dropped.

# 5. Maintenance-Related Constraints

- LiDAR window must be cleaned every 40 operating hours in environments with >1 mg/m³ particulate density; soiled windows cause detection range reduction.

- Wheel drive modules require gearbox oil inspection at 2000 hours; sealed maintenance-free option available for food-grade environments.

- Battery capacity below 80% SoH triggers reduced-speed mode; replacement is recommended below 70% SoH.