SYSTEM SCHEMATIC & SIGNAL FLOW

AGV-X200

Automatic Guided Vehicle (AGV-X200)

Version 1.0  |  2025

Industrial Robotics Division

Note: This document presents textual/tabular schematic descriptions suitable for RAG model ingestion. Graphical CAD schematics are available as separate DXF/PDF artefacts.

# 1. Top-Level System Block Diagram

The AGV-X200 is organised into five primary subsystems: (1) Power, (2) Compute & Control, (3) Navigation Sensing, (4) Drive & Motion, and (5) Communication. All subsystems are interconnected via a central power distribution board and a CAN/Ethernet internal bus.

# 2. Power Flow Schematic

## 2.1 Power Distribution

| Source | Converter/Path | Load |
| --- | --- | --- |
| AC Mains 230V | Charger → BMS | LiFePO4 Battery 48V/100Ah |
| Battery 48V | Direct | Left Hub Motor Driver (BLDC) |
| Battery 48V | Direct | Right Hub Motor Driver (BLDC) |
| Battery 48V | DC-DC 48→24V (10A) | Safety PLC, Sensors, Actuators |
| Battery 48V | DC-DC 48→12V (5A) | Wi-Fi Module, CAN Bus, I/O |
| Battery 48V | DC-DC 48→5V (3A) | Compute SBC, USB Peripherals |
| 24V Bus | Direct | LiDAR, Ultrasonic Array, Bumper |
| BMS | Cell balancing/protection | Battery pack cells (series-parallel) |

# 3. Signal & Data Flow Schematic

## 3.1 Internal Communication Bus

| Bus | Nodes Connected | Protocol / Rate |
| --- | --- | --- |
| CAN 2.0B (1 Mbps) | Compute SBC ↔ Motor Driver L/R | CANOpen DS402 |
| CAN 2.0B (1 Mbps) | Compute SBC ↔ Safety PLC | CANOpen Safety |
| Ethernet (100Mbps) | Compute SBC ↔ LiDAR (primary) | UDP ROS2 topic |
| USB 3.0 | Compute SBC ↔ LiDAR (backup) | RPLIDAR USB driver |
| I2C (400 kHz) | Compute SBC ↔ BMS | SMBus telemetry |
| GPIO (24V) | Safety PLC ↔ E-Stop buttons | Dual-channel hardwired |
| GPIO (24V) | Safety PLC ↔ Safety Laser Scanner | OSSD dual-channel |
| GPIO (24V) | Safety PLC ↔ Motor Enable | STO signal to drivers |

# 4. External Interface Schematic

## 4.1 External Connections Summary

| Anderson SB120 (rear panel) | Battery charging / hot-swap interface |
| --- | --- |
| RJ45 (GbE, isolated) | Dockside data transfer, engineering access |
| Wi-Fi 6 antenna × 2 | Fleet management, mission orders, telemetry |
| E-Stop push button × 2 | Manual emergency stop (front/rear) |
| Safety I/O 14-pin M12 | External safety light curtain or area scanner |
| Payload 24V / CAN terminal | Power and data to top deck attachment modules |
| USB-A × 2 (IP54 sealed) | Diagnostics laptop, map import/export |
| Digital I/O (8 × 24V) | Conveyor handshake, door opener, signal tower |

# 5. Safety Architecture Schematic

The AGV-X200 safety architecture follows a two-channel (Cat. 3, PLd) design. Both E-Stop channels are hardwired to the Pilz PNOZ-m safety relay, which controls the Motor Enable relay coil. A software watchdog on the ROS2 Nav2 stack independently monitors all safety zones; timeout triggers the same relay chain. OSSD outputs from the safety scanner are wired as an AND function with the E-Stop chain.