TECHNICAL SPECIFICATION

AGV-X200

Automatic Guided Vehicle (AGV-X200)

Version 1.0  |  2025

Industrial Robotics Division

# 1. Scope & Purpose

This specification defines the functional, performance, mechanical, electrical, and software requirements for the Automatic Guided Vehicle (AGV-X200). It serves as the authoritative reference for procurement, integration engineering, and acceptance testing.

# 2. Performance Specifications

## 2.1 Navigation Performance

| Parameter | Specification | Test Method |
| --- | --- | --- |
| Static localisation accuracy | ±10 mm (2σ, open area) | ISO 3691-4 Annex A |
| Dynamic localisation accuracy | ±25 mm at 1.5 m/s | Internal test loop |
| Map building time | < 15 min per 1000 m² | Factory acceptance test |
| Re-localisation time | < 8 s after recovery | Controlled power-cycle test |
| Path following error | < ±20 mm (steady-state) | Straight 20 m corridor test |

## 2.2 Motion Performance

| Parameter | Specification | Test Method |
| --- | --- | --- |
| Maximum speed (unloaded) | 2.5 m/s | Flat surface, no obstacles |
| Maximum speed (500 kg load) | 1.8 m/s | Flat surface, rated load |
| Maximum acceleration | 0.5 m/s² | All loading conditions |
| Emergency stop distance (1.5 m/s) | < 350 mm | ISO 3691-4 §5.11 |
| Gradeability | 5° (rated load) | Incline test rig |

## 2.3 Energy Performance

| Parameter | Specification | Test Method |
| --- | --- | --- |
| Operating endurance (rated load) | ≥ 10 hours | Continuous duty cycle test |
| Standby endurance | ≥ 72 hours | Sleep mode, no motion |
| Fast charge time (20–80%) | < 60 minutes | Bench charger test |
| Full charge time (0–100%) | < 2 hours (fast) / < 6 hours (std) | Bench charger test |
| Regenerative braking recovery | ≥ 8% energy recovery | Controlled decel cycle |

# 3. Electrical Specifications

| Nominal voltage | 48V DC |
| --- | --- |
| Battery capacity | 100 Ah (4.8 kWh) |
| Peak current draw | 120 A (acceleration) |
| Standby current draw | 2 A |
| 24V logic supply | 10 A continuous |
| Charging input | 200–240V AC, 50/60 Hz, 16A |
| Electrical protection class | Class III (SELV) |
| Insulation resistance | > 10 MΩ (battery isolated) |

# 4. Software & Interface Specifications

- Operating system: Ubuntu 22.04 LTS (ROS2 Humble runtime)

- Fleet API: REST (JSON/HTTPS) and MQTT 3.1.1; VDA 5050 v1.1 order interface

- Navigation stack: ROS2 Nav2 with custom AGV path smoother plugin

- Map format: PGM + YAML (ROS2 map_server compatible)

- Logging: ROS2 bag (.mcap format), configurable topic list, auto-upload on dock

- OTA update: Signed firmware bundles, atomic update with verified rollback

- Simulation: Gazebo plugin available; ROS2 bridge for digital twin integration

# 5. Compliance & Standards

| ISO 3691-4:2020 | Industrial trucks — AGV safety requirements |
| --- | --- |
| IEC 61508 | Functional safety — SIL 2 navigation stop function |
| ISO 13849-1 | Safety-related control systems — PLd category 3 |
| UL 3100 | Outline of investigation for AGVs (North America) |
| CE (Machinery Directive) | 2006/42/EC compliance |
| FCC Part 15 / ETSI EN 300 328 | Wi-Fi radio compliance |
| UN 38.3 | Battery transport safety |
| RoHS 3 (2015/863/EU) | Hazardous substances restriction |