USAGE, DEPLOYMENT & OPERATING GUIDE

AGV-X200

Automatic Guided Vehicle (AGV-X200)

Version 1.0  |  2025

Industrial Robotics Division

# 1. Safety Precautions

Read this section in full before operating the system. Failure to observe safety requirements may result in personal injury, equipment damage, or production loss.

- Always verify the safety scanner field configuration matches the site layout before commissioning.

- Never disable or bypass the hardware E-Stop circuit for any reason during live operation.

- Ensure floor markings and pedestrian segregation barriers are in place before enabling autonomous navigation.

- Do not exceed rated payload; verify load centre-of-gravity is within ±100 mm of the deck centreline.

- Battery hot-swap must only be performed with the vehicle stationary and in E-Stop state.

# 2. Pre-Operation Checklist

- Verify battery state-of-charge ≥ 20% before dispatch.

- Inspect all four wheels/casters for debris, wear, and secure fastening.

- Confirm LiDAR lens is clean; wipe with lint-free cloth if required.

- Test E-Stop buttons (front and rear); verify vehicle halts within 350 mm.

- Load the correct site map in the navigation controller and verify localisation quality ≥ 80%.

- Confirm Wi-Fi signal strength in all planned operating zones (RSSI ≥ -65 dBm).

- Verify fleet management system shows vehicle as 'ready' and mission queue is empty.

- Perform a supervised manual drive test around the full planned route before enabling autonomous mode.

# 3. Deployment Procedure

## 3.1 Site Survey & Map Building

Before deployment, conduct a site survey to identify potential LiDAR reflection anomalies (glass walls, retroreflective tape). Mark all required QR waypoints at key decision points.

- Place the AGV in the designated map-building start location.

- Enable SLAM mapping mode via fleet management UI.

- Manually drive the AGV at low speed (<0.5 m/s) along all planned routes.

- Return to start location; save and export map as PGM+YAML.

- Review map for completeness; add no-go zones and speed zones via map editor.

- Upload finalised map to all vehicles in the fleet.

## 3.2 Mission Programming

- Open the fleet management web UI (default: https://agv-fleet.local:8080).

- Navigate to 'Nodes' and define waypoints at all pick/drop locations.

- Create route edges connecting nodes; assign speed limits and priority.

- Define mission templates: pick, drop, charge, park.

- Assign missions to the vehicle; verify execution in simulation mode first.

- Monitor first live mission from the dashboard; confirm cycle time meets target.

# 4. Normal Operation

During normal autonomous operation, the AGV requires no operator intervention. The operator's role is monitoring and exception handling:

- Monitor vehicle status on fleet dashboard (speed, battery, mission state).

- Respond to 'blocked path' alerts by investigating and clearing physical obstacles.

- Respond to 'map update required' alerts after any permanent layout change.

- Review daily operation log for repeated navigation recoveries at specific locations.

- Schedule battery swap or dock charging before SoC reaches 20%.

# 5. Maintenance Schedule

| Task | Interval | Personnel |
| --- | --- | --- |
| LiDAR lens cleaning | 40 operating hours | Operator |
| Wheel wear inspection | 200 hours | Maintenance Tech |
| Gearbox oil level check | 500 hours | Maintenance Tech |
| Battery SoH test | 1000 hours or 6 months | Maintenance Tech |
| Safety scanner recalibration | 6 months | Safety Engineer |
| Full map re-survey | Annually or after layout change | Commissioning Engineer |
| Firmware update review | Quarterly | System Integrator |
| Mechanical inspection (fasteners, cables) | 1000 hours | Maintenance Tech |

# 6. Troubleshooting Quick Reference

| Symptom | Probable Cause | Recommended Action |
| --- | --- | --- |
| Vehicle does not localise after startup | Map mismatch / LiDAR blocked | Clean LiDAR lens; verify map file version |
| Repeated path blocked alerts at one location | Permanent new obstacle in map | Update map and no-go zone |
| Battery SoC drops faster than expected | Cell ageing or temperature effect | Run BMS health report; replace if SoH < 80% |
| Wi-Fi dropout in specific zone | Coverage gap or interference | Add AP or change channel assignment |
| E-Stop triggered without visible obstacle | Safety scanner misalignment or dirt | Clean scanner; verify field configuration |
| Vehicle drifts off expected path | Wheel wear or encoder fault | Inspect drive wheels; check encoder cable |
| Mission stuck in 'waiting' state | Broker connectivity loss | Check MQTT broker status; restart service |
| Charging dock fails to connect | Misalignment or connector fouling | Clean charging contacts; verify dock alignment |

# 7. Decommissioning & Disposal

- Discharge battery to <5% SoC before removal.

- LiFePO4 batteries must be disposed of via certified battery recycling; do not landfill.

- LiDAR contains laser class 1M components; dispose in compliance with WEEE directive.

- Frame and motors may be recycled as industrial scrap metal.