AUTONOMOUS MOBILE ROBOT

KNOWLEDGE BASE

3. SPECIFICATION FILE

Technical Specifications — All Subsystems

Date: June 2025  |  Version: 1.0  |  Status: Active

# 1. Platform Specifications

| Parameter | Value | Unit | Notes |
| --- | --- | --- | --- |
| Overall Length | 800 | mm | Including bumpers |
| Overall Width | 600 | mm | — |
| Overall Height | 450 | mm | Antenna excluded |
| Weight (unladen) | 28 | kg | Without payload |
| Max Payload | 50 | kg | On flat surface |
| Ground Clearance | 40 | mm | Minimum |
| Turning Radius | 0 (in-place) | — | Differential drive |
| Max Speed | 1.5 | m/s | Loaded condition |
| Max Speed (unladen) | 2.0 | m/s | No payload |
| Slope Capability | 10 | deg | Max incline |
| Operating Temperature | -10 to +50 | °C | — |
| IP Rating | IP54 | — | Dust + splash resistant |

## 2. Drive & Locomotion Specifications

| Parameter | Value | Unit |
| --- | --- | --- |
| Drive Type | Differential Drive | — |
| Motor Model | Maxon EC-i 40 | — |
| Rated Torque (per motor) | 68 | mNm |
| Peak Torque (per motor) | 204 | mNm |
| Rated Speed | 4000 | RPM |
| Gear Ratio | 14:1 | — |
| Encoder Resolution | 4096 | CPR (quadrature) |
| Wheel Diameter | 200 | mm |
| Wheel Width | 50 | mm |
| Wheelbase | 400 | mm |
| Motor Controller | ODrive v3.6 | — |
| Control Mode | Field-Oriented Control (FOC) | — |

## 3. Power System Specifications

| Parameter | Value | Unit |
| --- | --- | --- |
| Battery Chemistry | LiFePO4 | — |
| Nominal Voltage | 24 | V |
| Capacity | 50 | Ah |
| Energy | 1200 | Wh |
| Max Continuous Discharge | 60 | A |
| Peak Discharge | 100 | A (10s) |
| Charging Voltage | 29.2 | V |
| BMS Cell Balancing | Active | — |
| Estimated Runtime (full load) | 4 | hours |
| Estimated Runtime (light load) | 8 | hours |
| Low Battery Threshold | 21.0 | V |
| Charging Time (0–100%) | 3 | hours |

## 4. Sensing Specifications

| Sensor | Specification | Value |
| --- | --- | --- |
| LiDAR (RPLIDAR A3) | Range | 0.2 – 25 m |
| LiDAR (RPLIDAR A3) | Angular Resolution | 0.33° @ 10 Hz |
| LiDAR (RPLIDAR A3) | Sample Rate | 16,000 samples/s |
| Depth Camera (D435i) | Depth Range | 0.3 – 3 m |
| Depth Camera (D435i) | Depth Resolution | 1280 × 720 px |
| Depth Camera (D435i) | Frame Rate | 90 fps |
| IMU (BNO055) | Accelerometer Range | ±16 g |
| IMU (BNO055) | Gyroscope Range | ±2000 °/s |
| IMU (BNO055) | Heading Accuracy | ± 2.5° (absolute) |
| Ultrasonic (HC-SR04) | Range | 0.02 – 4 m |
| Ultrasonic (HC-SR04) | Beam Angle | 15° |

## 5. Communication & Compute Specifications

| Subsystem | Specification | Value |
| --- | --- | --- |
| Main Computer | SBC | Raspberry Pi 4B — 8 GB RAM |
| Real-Time Controller | MCU | STM32F407 @ 168 MHz |
| AI Accelerator | GPU/NPU | NVIDIA Jetson Nano — 128-core Maxwell |
| WiFi | Standard | IEEE 802.11ax (WiFi 6) |
| Ethernet | Speed | Gigabit (1000BASE-T) |
| CAN Bus | Protocol | CAN 2.0B @ 1 Mbit/s |
| USB | Version | USB 3.0 Hub — 7 port |
| Operating System | ROS | ROS 2 Humble (Ubuntu 22.04) |
| Navigation Stack | Framework | Nav2 with SLAM Toolbox |
| Localization | Method | EKF Sensor Fusion (LiDAR + IMU + Odometry) |

## 6. Safety & Compliance Specifications

| Requirement | Standard / Value | Status |
| --- | --- | --- |
| Functional Safety | IEC 61508 SIL-2 | In Progress |
| AMR Safety | ISO 3691-4 | In Progress |
| EMC | CE / FCC Part 15B | Pending |
| Battery Safety | UN 38.3 / IEC 62133 | Certified |
| E-Stop Response Time | < 100 ms | Verified |
| Obstacle Detection Range | 0.3 m (safety zone) | Configured |
| Warning Zone | 0.8 m | Configured |
| Protective Field Shape | Adjustable — 3 zones | Configurable |