AUTONOMOUS MOBILE ROBOT

KNOWLEDGE BASE

4. COMPONENT MAPPING

Interface, Connectivity & System Integration Map

Date: June 2025  |  Version: 1.0  |  Status: Active

# 1. Overview

This document defines how all components in the AMR-X1 are interconnected. It covers electrical interfaces, data buses, power rails, and communication protocols between every subsystem.

## 2. Power Rail Mapping

| Power Rail | Voltage | Source | Consumers | Max Current |
| --- | --- | --- | --- | --- |
| VBAT | 24V (nominal) | LiFePO4 Battery | Motor drivers, DC-DC converters | 60A continuous |
| VCC_12V | 12V | Pololu D24V150F12 | LiDAR, Jetson Nano, Fans | 15A |
| VCC_5V | 5V | Pololu D24V150F5 | RPi 4B, USB Hub, IMU, Ultrasonics | 15A |
| VCC_3V3 | 3.3V | STM32 onboard LDO | MCU peripherals, CAN transceiver | 500mA |

## 3. Data Interface Mapping

| From Component | To Component | Interface | Protocol/Speed | Notes |
| --- | --- | --- | --- | --- |
| RPi 4B | STM32F407 | USB Serial / UART | 115200 baud | ROS 2 micro-ROS bridge |
| RPi 4B | Intel AX200 (WiFi) | PCIe / M.2 | PCIe Gen3 | WiFi 6 card |
| RPi 4B | USB Hub | USB 3.0 | 5 Gbps | Central USB distribution |
| USB Hub | RPLIDAR A3 | USB 2.0 | 4 Mbps | Serial data |
| USB Hub | RealSense D435i | USB 3.0 | 5 Gbps | Depth stream |
| USB Hub | Jetson Nano | USB 3.0 | 5 Gbps | AI inference results |
| STM32F407 | ODrive v3.6 | UART / CAN | CAN @ 1Mbps | Motor commands + encoder feedback |
| STM32F407 | BNO055 IMU | I2C | 400 kHz (Fast Mode) | Sensor data @ 100 Hz |
| STM32F407 | HC-SR04 x4 | GPIO (Trigger/Echo) | Digital pulse | Ultrasonic obstacle detect |
| STM32F407 | E-Stop Module | GPIO | Digital input | Emergency stop signal |
| ODrive v3.6 | Motor L + Encoder L | 3-phase PWM + Quad Enc | — | Left drive axis |
| ODrive v3.6 | Motor R + Encoder R | 3-phase PWM + Quad Enc | — | Right drive axis |
| Jetson Nano | RealSense D435i | USB 3.0 | 5 Gbps | AI vision pipeline |
| BMS | Battery Pack | Direct | — | Cell monitoring + protection |
| BMS | STM32F407 | UART / I2C | 9600 baud | SOC + fault reporting |

## 4. CAN Bus Node Map

| CAN Node ID | Device | Role | Baud Rate |
| --- | --- | --- | --- |
| 0x01 | ODrive v3.6 (Motor L) | Motor axis 0 control | 1 Mbit/s |
| 0x02 | ODrive v3.6 (Motor R) | Motor axis 1 control | 1 Mbit/s |
| 0x10 | STM32F407 (Master) | CAN master — command + telemetry | 1 Mbit/s |
| 0x20 | BMS | Battery status broadcast | 1 Mbit/s |
| 0x30 | E-Stop Module | Safety watchdog heartbeat | 1 Mbit/s |

## 5. I2C Bus Mapping (STM32 I2C1)

| I2C Address | Device | Data | Speed |
| --- | --- | --- | --- |
| 0x28 | BNO055 IMU (Primary) | Accel / Gyro / Fusion @ 100 Hz | 400 kHz |
| 0x29 | BNO055 IMU (Alt addr) | Spare / not populated | 400 kHz |
| 0x68 | MPU-6050 (Backup IMU) | Raw accel + gyro (redundancy) | 400 kHz |
| 0x48 | ADS1115 ADC | Battery voltage monitor (4-ch) | 400 kHz |

## 6. GPIO Pin Mapping — STM32F407

| STM32 Pin | Function | Connected To | Direction |
| --- | --- | --- | --- |
| PA0 (TIM2_CH1) | PWM — Trigger HC-SR04 #1 | Ultrasonic Front-Left | Output |
| PA1 (TIM2_CH2) | Echo — HC-SR04 #1 | Ultrasonic Front-Left | Input |
| PA2 (TIM2_CH3) | PWM — Trigger HC-SR04 #2 | Ultrasonic Front-Right | Output |
| PA3 (TIM2_CH4) | Echo — HC-SR04 #2 | Ultrasonic Front-Right | Input |
| PB4 (TIM3_CH1) | PWM — Trigger HC-SR04 #3 | Ultrasonic Rear-Left | Output |
| PB5 (TIM3_CH2) | Echo — HC-SR04 #3 | Ultrasonic Rear-Left | Input |
| PB6 (TIM3_CH3) | PWM — Trigger HC-SR04 #4 | Ultrasonic Rear-Right | Output |
| PB7 (TIM3_CH4) | Echo — HC-SR04 #4 | Ultrasonic Rear-Right | Input |
| PC13 | E-Stop Input (active LOW) | E-Stop Module Relay Out | Input |
| PC14 | Status LED — Green | Front panel LED | Output |
| PC15 | Status LED — Red | Front panel LED | Output |
| PB10 (USART3_TX) | UART TX to RPi | RPi 4B GPIO14 (RX) | Output |
| PB11 (USART3_RX) | UART RX from RPi | RPi 4B GPIO15 (TX) | Input |

## 7. Network Topology

The AMR communicates with a central Fleet Management Server (FMS) via WiFi 6. Internal communication uses ROS 2 DDS (FastDDS) on a local Ethernet switch. The Jetson Nano connects to the RPi via USB 3.0 for inference result streaming.

| Node | IP Address | Role | Protocol |
| --- | --- | --- | --- |
| RPi 4B (Main) | 192.168.1.10 | ROS 2 Master + Navigation | ROS 2 / DDS |
| Jetson Nano | 192.168.1.11 | AI Vision Inference | USB + ROS 2 topic |
| Fleet Server | 192.168.1.1 | Mission planning + monitoring | MQTT + REST API |
| Operator Tablet | DHCP | Manual control + map viewing | MQTT / WebSocket |