AUTONOMOUS MOBILE ROBOT

KNOWLEDGE BASE

5. SCHEMATICS

Electrical & Wiring Schematics Index & Reference

Date: June 2025  |  Version: 1.0  |  Status: Active

# 1. Overview

This document serves as the master index and reference guide for all electrical schematics associated with the AMR-X1 platform. It describes what each schematic contains, the file naming convention, and the collection/upload workflow for circuit diagrams, wiring diagrams, and PCB layouts.

## 2. Schematics Index

| Schematic ID | Title | Description | File Name | Format |
| --- | --- | --- | --- | --- |
| SCH-001 | Main Power Distribution | 24V battery → BMS → fuses → DC-DC rails → loads | Power_Custom_MainPowerDist_Schematic.pdf | PDF + KiCad |
| SCH-002 | Motor Drive Circuit | ODrive v3.6 connections, phase wiring, shunt resistors | Motor_ODrive_v36_DriveCircuit_Schematic.pdf | PDF |
| SCH-003 | STM32 Core Board | MCU pin connections, decoupling caps, crystal, JTAG | Controller_STM_STM32F407_CoreBoard_Schematic.pdf | PDF + KiCad |
| SCH-004 | Sensor Interface PCB | IMU, ultrasonic trigger/echo, I2C pull-ups, level shifters | Sensor_Custom_SensorInterfacePCB_Schematic.pdf | PDF + KiCad |
| SCH-005 | E-Stop & Safety Circuit | Relay logic, watchdog timer, manual E-stop wiring | Safety_Custom_EStop_Schematic.pdf | PDF |
| SCH-006 | CAN Bus Harness | STM32 ↔ ODrive CAN wiring, termination resistors, topology | Comms_Custom_CANBus_Wiring.pdf | PDF |
| SCH-007 | Power Harness Diagram | Physical wire routing, gauge, connector pin-out | Power_Custom_PowerHarness_Wiring.pdf | PDF |
| SCH-008 | LiDAR Wiring | RPLIDAR A3 USB + 12V power connection | Sensor_Slamtec_RPLIDARA3_Wiring.pdf | PDF |
| SCH-009 | Compute Stack Wiring | RPi4B ↔ Jetson ↔ USB hub interconnections | Computer_Custom_ComputeStack_Wiring.pdf | PDF |
| SCH-010 | BMS Integration | Daly BMS cell wiring, UART to STM32, balancing circuit | Power_Daly_BMS24V_Integration_Schematic.pdf | PDF |

## 3. Schematic Naming Convention

Category_Manufacturer_Component_DocumentType.pdf

For schematics, the DocumentType field should be one of:

- Schematic — Full circuit diagram

- Wiring — Physical wiring diagram or harness layout

- PCB — PCB layout file or Gerber export

- Pinout — Pin assignment diagram only

## 4. Pinout Reference Table

### 4.1 STM32F407 Key Connectors

| Connector | Pin | Signal | Direction | Notes |
| --- | --- | --- | --- | --- |
| J1 (Power) | 1 | VCC 3.3V | Power In | From LDO |
| J1 (Power) | 2 | GND | Ground | — |
| J2 (UART to RPi) | 1 | TX (PA9) | Output | 3.3V logic |
| J2 (UART to RPi) | 2 | RX (PA10) | Input | 3.3V logic |
| J2 (UART to RPi) | 3 | GND | Ground | Common GND |
| J3 (CAN Bus) | 1 | CANH | Diff | 120Ω terminated |
| J3 (CAN Bus) | 2 | CANL | Diff | 120Ω terminated |
| J3 (CAN Bus) | 3 | GND | Ground | — |
| J4 (I2C) | 1 | SCL (PB6) | Output | 4.7kΩ pull-up |
| J4 (I2C) | 2 | SDA (PB7) | Bidir | 4.7kΩ pull-up |
| J4 (I2C) | 3 | VCC 3.3V | Power Out | IMU supply |
| J4 (I2C) | 4 | GND | Ground | — |

### 4.2 ODrive v3.6 Connector Summary

| Connector | Signal | Description |
| --- | --- | --- |
| M0 Phase A/B/C | Motor L phases | Left drive motor — 3-phase brushless |
| M1 Phase A/B/C | Motor R phases | Right drive motor — 3-phase brushless |
| ENC0 A/B/Z | Encoder L | Quadrature encoder left — 5V |
| ENC1 A/B/Z | Encoder R | Quadrature encoder right — 5V |
| CAN H/L | CAN Bus | Connects to STM32 CAN network |
| DC+/DC- | 24V Power Input | From main power distribution |
| 3.3V / 5V Rail | Aux Power Out | For encoder VCC supply |

## 5. Wiring Diagram Standards

| Wire Gauge | Application | Color Code | Max Current |
| --- | --- | --- | --- |
| 10 AWG | Main battery to BMS | Red (+) / Black (−) | 60A |
| 12 AWG | BMS to motor drivers | Orange | 40A |
| 16 AWG | DC-DC converter outputs (12V rail) | Yellow | 15A |
| 18 AWG | DC-DC converter outputs (5V rail) | Red | 15A |
| 22 AWG | Signal wiring (CAN, UART, I2C) | Various / per IEC 60757 | 500mA |
| 24 AWG | Sensor wiring (ultrasonic, LED) | White | 200mA |
| 28 AWG | Board-to-board ribbon cables | Gray | 100mA |

## 6. Schematic Collection Workflow

For every schematic file collected or created, follow this workflow:

- Download or export schematic from the original EDA tool (KiCad, Altium, Eagle)

- Export to PDF — include schematic title block with revision and date

- Rename using the standard format: Category_Manufacturer_Component_DocumentType.pdf

- Upload to the Shared Drive → /Schematics/ folder

- Add an entry to the Notion database with all required metadata fields

- Record the schematic ID (SCH-XXX) in the component registry

## 7. Schematic Software Used

| Tool | Version | Used For | File Format |
| --- | --- | --- | --- |
| KiCad | 7.0 | Custom PCB schematics (Sensor Interface, STM32 board) | .kicad_sch / .kicad_pcb |
| Fritzing | 0.9.10 | Simple wiring diagrams for documentation | .fzz / .png |
| draw.io | Latest (web) | Block-level wiring diagrams | .drawio / .pdf |
| LTspice XVII | XVII | Power circuit simulation | .asc / .raw |