AUTONOMOUS MOBILE ROBOT

KNOWLEDGE BASE

6. CAD MODEL

CAD Files, STEP Models & Mechanical Design Reference

Date: June 2025  |  Version: 1.0  |  Status: Active

# 1. Overview

This document is the master index for all CAD models, STEP files, drawings, and mechanical design assets associated with the AMR-X1 platform. It covers the complete mechanical structure, component enclosures, mounting hardware, and cable management geometry.

## 2. CAD File Index

| CAD ID | Assembly / Part | Description | File Name | Format |
| --- | --- | --- | --- | --- |
| CAD-001 | AMR-X1 Complete Assembly | Full robot — all subsystems assembled | Robot_Internal_AMRX1_FullAssembly.step | STEP + SOLIDWORKS |
| CAD-002 | Chassis Frame | Welded aluminum 6061-T6 frame | Mechanical_Custom_ChassisFrame.step | STEP |
| CAD-003 | Drive Wheel Assembly | 200mm PU wheel + hub + motor mount | Mechanical_Custom_DriveWheelAssy.step | STEP |
| CAD-004 | Caster Wheel Mount | 75mm swivel caster bracket + mounting plate | Mechanical_Custom_CasterMount.step | STEP |
| CAD-005 | LiDAR Tower | RPLIDAR A3 elevation tower with cable routing | Mechanical_Custom_LiDARTower.step | STEP |
| CAD-006 | Electronics Enclosure | Sealed IP54 enclosure — compute stack + PCBs | Mechanical_Custom_ElectronicsBox.step | STEP |
| CAD-007 | Battery Tray | LiFePO4 pack tray with retention straps | Mechanical_Custom_BatteryTray.step | STEP |
| CAD-008 | Depth Camera Bracket | RealSense D435i forward-facing bracket | Mechanical_Custom_CameraBracket.step | STEP |
| CAD-009 | Ultrasonic Sensor Mounts | 4× HC-SR04 front and rear mounting bosses | Mechanical_Custom_UltrasonicMounts.step | STEP |
| CAD-010 | T-Slot Rail Frame (Top) | Misumi HFS5-3030 top payload deck frame | Mechanical_Misumi_TopDeck.step | STEP |
| CAD-011 | Motor Mount Plate | Maxon EC-i 40 precision motor mount | Mechanical_Custom_MotorMount.step | STEP |
| CAD-012 | Cable Tray | Internal cable management routing trays | Mechanical_Custom_CableTray.step | STEP |
| CAD-013 | Bumper System | Front + rear foam-core bumper with brackets | Mechanical_Custom_BumperSystem.step | STEP |
| CAD-014 | E-Stop Button Panel | External emergency stop push-button panel | Mechanical_Custom_EStopPanel.step | STEP |

## 3. CAD File Naming Convention

Category_Manufacturer_Component_DocumentType.step

DocumentType for CAD files should be one of:

- (no suffix required for STEP) — e.g., Mechanical_Custom_ChassisFrame.step

- _Assembly — complete multi-part assembly STEP

- _Drawing — 2D engineering drawing (PDF)

- _BOM — Bill of Materials for the assembly

## 4. CAD Software & Formats

| Software | Version | File Format | Used For |
| --- | --- | --- | --- |
| SOLIDWORKS | 2024 SP2 | .sldprt / .sldasm | Primary mechanical design tool |
| STEP (ISO 10303) | AP214 | .step / .stp | Universal exchange format — required for all parts |
| Autodesk Fusion 360 | Latest | .f3d / .step | Secondary modeling + CAM for machined parts |
| FreeCAD | 0.21 | .FCStd / .step | Open-source — used for community contributions |
| STL | — | .stl | 3D printing of brackets / enclosures only |
| DXF | R2018 | .dxf | 2D profiles for laser cut sheet metal parts |

## 5. Key Mechanical Parameters

| Parameter | Value | Notes |
| --- | --- | --- |
| Chassis Material | 6061-T6 Aluminum | Welded + powder coated |
| Sheet Metal Thickness | 3 mm | Electronics enclosure panels |
| T-Slot Profile | 30 × 30 mm (Misumi HFS5-3030) | Top deck payload rail |
| Motor Mount Tolerance | H7/g6 shaft fit | Precision bore for Maxon EC-i 40 |
| Wheel Axle Diameter | 12 mm | Hardened steel shaft |
| Bearing Type | 6001-2RS deep groove | Wheel hubs |
| Fasteners | M4, M5, M6 ISO 4762 (hex socket) | All structural fasteners |
| Loctite Grade | 243 (medium strength) | All critical fastener threads |
| Enclosure IP Rating | IP54 | Gasket + captive screws |
| Max Allowable Deflection | 2 mm at 50 kg payload midspan | Chassis bending requirement |

## 6. Bill of Materials — Structural (Top Level)

| Item | Description | Qty | Material | Supplier | Part Number |
| --- | --- | --- | --- | --- | --- |
| 1 | Main chassis frame | 1 | Al 6061-T6 | Custom | CHASSIS-REV3 |
| 2 | Drive motor mount plate | 2 | Al 6061-T6 | Custom | MOTMNT-REV2 |
| 3 | Drive wheel (200mm PU) | 2 | PU / Steel hub | Guitel | GU-200-PU-12 |
| 4 | Swivel caster 75mm | 2 | Steel / Nylon | Shepherd | SH-75-SW-6 |
| 5 | T-Slot rail 30×30 × 560mm | 4 | Al 6063 | Misumi | HFS5-3030-560 |
| 6 | T-Slot rail 30×30 × 400mm | 2 | Al 6063 | Misumi | HFS5-3030-400 |
| 7 | Electronics enclosure box | 1 | Al sheet 3mm | Custom | ENCL-IP54-REV2 |
| 8 | Battery tray assembly | 1 | Al 6061-T6 | Custom | BATT-TRAY-REV1 |
| 9 | LiDAR tower bracket | 1 | Al 6061-T6 | Custom | LIDAR-TWR-REV2 |
| 10 | Camera mount bracket | 1 | Al 6061-T6 | Custom | CAM-MNT-REV1 |

## 7. CAD Collection Workflow

Follow these steps for every CAD file collected from a vendor or created internally:

- Download official STEP file from manufacturer or export from CAD software

- Verify STEP file opens correctly and geometry is complete (no missing faces)

- Apply standard coordinate origin — robot center at ground level, X forward

- Rename file using the standard convention

- Upload to Shared Drive → /CAD/STEP/ folder

- Update Notion database entry with file details and source URL

- Reference the CAD-ID in the component registry and specification file

## 8. Approved Vendor CAD Sources

| Component | CAD Source URL | Format Available |
| --- | --- | --- |
| Maxon EC-i 40 Motor | maxongroup.com → CAD downloads | STEP, DXF |
| RPLIDAR A3 | slamtec.com → resources | STEP |
| RealSense D435i | github.com/IntelRealSense/librealsense | STEP, SolidWorks |
| BNO055 IMU | bosch-sensortec.com → design resources | STEP (PCB footprint) |
| Misumi T-Slot Profiles | misumi-ec.com → 3D CAD | STEP, IGES, DXF |
| Pololu Voltage Regulators | pololu.com → resources → 3D files | STEP |
| ODrive v3.6 | github.com/odriverobotics/ODrive | STEP, Altium |
| Raspberry Pi 4B | grabcad.com / raspberrypi.com | STEP |
| Jetson Nano | grabcad.com / developer.nvidia.com | STEP |