# Control Algorithms

PID, Motion Control, Path Planning & Calibration

## 1. PID Control

PID controllers form the innermost control loop for each axis. Tuning parameters (Kp, Ki, Kd) are typically set via auto-tuning routines in commercial drives or manually using Ziegler-Nichols or SIMC rules.

| Parameter | Effect & Tuning Guidance |
| --- | --- |
| Proportional Gain (Kp) | Increases stiffness; too high → oscillation |
| Integral Gain (Ki) | Eliminates steady-state error; too high → windup |
| Derivative Gain (Kd) | Damps overshoot; filter derivative to reduce noise sensitivity |
| Feed-Forward (Kff) | Improves tracking by predicting required torque |
| Anti-Windup | Clamp integrator when output saturated |
| Sample Rate | 1 kHz minimum; 4–8 kHz for servo loops |

## 2. Motion Control

- Trapezoidal velocity profile: constant acceleration → constant velocity → deceleration

- S-curve (sinusoidal) profile: jerk-limited for smooth motion and reduced vibration

- Electronic cam: synchronised multi-axis motion using cam table lookup

- Electronic gearing: slave axis tracks master position × gear ratio

- Torque feed-forward: gravity compensation using dynamic model parameters

## 3. Path Planning Algorithms

| Algorithm | Type | Use Case | Key Property |
| --- | --- | --- | --- |
| RRT (Rapidly-exploring Random Tree) | Sampling-based | Joint space | Fast for high DOF |
| RRT* | Sampling-based | Joint / task space | Asymptotically optimal |
| A* Search | Graph-based | Grid workspace | Optimal on discrete grid |
| STOMP | Stochastic | Trajectory optimisation | Obstacle-aware smoothing |
| Cubic Spline | Deterministic | Via-point blending | C² continuity |

## 4. Calibration

Robot calibration corrects geometric and non-geometric errors between the kinematic model and the physical robot.

- Joint zero-offset calibration: align each axis to a reference mark with a dial indicator

- Link length identification: use a calibrated measurement artefact and least-squares optimisation

- TCP (Tool Centre Point) calibration: move TCP to a fixed point from 4+ orientations

- Payload identification: measure inertia and CoM of attached tool

- Full volumetric calibration: laser tracker measurement of 50+ poses across workspace

PID parameter files and calibration records are stored in 09_Control_Algorithms/Calibration/.