# Software

ROS, ROS2, MoveIt, Firmware & Simulations

## 1. ROS (Robot Operating System)

ROS Noetic (Ubuntu 20.04 LTS) is used for legacy integration. Core packages: ros_control, joint_state_publisher, robot_state_publisher, MoveIt 1.

| Package | Purpose |
| --- | --- |
| cartesian_robot_description | URDF / xacro robot model |
| cartesian_robot_moveit_config | MoveIt planning configuration |
| joint_trajectory_controller | Joint-space trajectory execution |
| ros_control hardware_interface | Real hardware abstraction layer |
| rqt_joint_trajectory_controller | GUI for manual joint jogging |

## 2. ROS2 (Humble / Iron)

- ROS2 Humble Hawksbill on Ubuntu 22.04 LTS — recommended for new projects

- DDS middleware: eProsima Fast DDS (default) or Cyclone DDS for real-time

- ros2_control: standardised hardware interface, controller manager, lifecycle management

- Nav2 (Cartesian mobile base): path planning, obstacle avoidance if mobile platform

- Micro-ROS: runs directly on STM32 for embedded joint control nodes

## 3. MoveIt 2

MoveIt 2 provides motion planning, collision checking, IK solving, and grasp planning.

| Feature | Description |
| --- | --- |
| Motion Planners | OMPL (RRT, RRTConnect, TRRT, STOMP, CHOMP) |
| Collision Objects | MoveIt CollisionEnvironment with OctoMap integration |
| IK Solvers | KDL (default), TRAC-IK, BioIK (optimisation-based) |
| Grasp Planning | MoveIt Task Constructor for multi-stage pick-and-place |
| Servo (Jog) | Real-time Cartesian jogging from SpaceMouse or game controller |

## 4. Firmware

- Joint controller firmware: FreeRTOS on STM32H7 — PID loop at 4 kHz, CAN FD communication

- Bootloader: STM32 USB DFU or custom CAN-based OTA update

- Safety watchdog: independent IWDG with 1 s timeout → E-stop output on MCU fault

- Flash memory layout: 0x0800 0000 bootloader, 0x0800 8000 application, 0x0810 0000 parameter store

## 5. Simulations

- Gazebo Classic / Gazebo Sim: physics simulation with ros_gz_bridge

- RViz2: 3D visualisation, trajectory display, TF tree

- MATLAB / Simulink Robotics Toolbox: kinematic & dynamic analysis, controller design

- PyBullet: lightweight Python-based simulation for algorithm prototyping

All source code is managed in Git repositories. Links and commit hashes are in 16_Metadata/sources.csv.