COMPONENT ARCHITECTURE DOCUMENT

MTR-S500

Machine Tending Robot (MTR-S500)

Version 1.0  |  2025

Industrial Robotics Division

# 1. System Overview

This document provides an architectural breakdown of the major subsystems and components of the Machine Tending Robot (MTR-S500). Each subsystem is described in terms of its function, interfaces, and integration dependencies.

# 2. Mechanical Subsystem

## 2.1 Robot Arm Structure

- 6 rotary joints (J1–J6): Aluminium housing with integrated harmonic drive reducers and hollow wrist for cable routing

- J1–J3 (body axes): Bearing-supported, high-torque harmonic drives; backlash < 1 arc-minute

- J4–J6 (wrist axes): Compact 3-axis wrist assembly; through-bore for pneumatic/electric end-effector cabling

- Base flange: ISO 9283 100 mm bolt circle for pedestal, wall, or ceiling mounting

- End-effector flange: ISO 9283 63 mm, mechanical key for repeatability on quick-change adaptor

## 2.2 End-Effector Module

- Quick-change coupler: Manual or automatic, rated 300N axial, 50 Nm torque

- Pneumatic parallel gripper: 80 mm stroke, 300 N grip force, dual-acting cylinder

- Electric adaptive gripper: 3-finger, 100 mm stroke, 150 N per finger, position-controlled

- Deburring spindle (optional): 30,000 RPM, ±3 N force-controlled compliance

# 3. Actuator & Drive Subsystem

- Joint actuators: AC servo motors with resolver feedback (23-bit absolute)

- Reducers: Harmonic drive (J1–J3) and planetary gearbox (J4–J6) for compact wrist

- Brake: Spring-applied, electrically-released electromagnetic brake on J1–J3

- Peak torque ratings: J1: 180 Nm, J2: 160 Nm, J3: 120 Nm, J4: 40 Nm, J5: 40 Nm, J6: 20 Nm

# 4. Sensing Subsystem

## 4.1 Proprioceptive

- Joint encoders: 23-bit absolute resolvers on each axis; fault-tolerant dual-channel

- 6-axis F/T sensor: Wrist-mounted, ±300 N / ±30 Nm, 1000 Hz sample rate, EtherCAT interface

- Motor current monitoring: Per-axis, 2 kHz, used for collision detection in base SKU

## 4.2 Exteroceptive

- 3D structured-light camera: 1.3 MP, 1.5 m range, 0.1 mm point cloud resolution, ROS2 driver

- 2D barcode scanner: QR/DataMatrix, 640×480, integrated LED ring light

- Optional 2D line-scan camera for in-line part quality inspection

# 5. Controller & Compute Subsystem

- Robot controller cabinet: 19-inch rack, 3U, IP54, steel housing

- Motion CPU: Intel Core i7 real-time processor running RT Linux + ROS2 control framework

- Safety CPU: Dual-channel TUV-certified safety controller, SIL 3 / PLe rated

- EtherCAT master: Beckhoff EL6695 master running at 1 ms cycle time

- HMI pendant: 10-inch touchscreen teach pendant with deadman grip; USB-C tether + wireless mode

# 6. Communication Subsystem

- EtherCAT: Primary real-time fieldbus; 1 ms deterministic cycle

- Profinet IO: For PLC integration in Siemens/Rockwell environments

- OPC-UA server: Telemetry read-out at 100 ms update rate

- REST API: Job dispatch, status query, configuration management over HTTPS

- MQTT client: Event publishing for MES/SCADA integration

# 7. Safety Subsystem

- Dual-channel hardware E-Stop: Hardwired to safety CPU, 40 ms response

- Safety-rated speed/position monitoring: SLS, SOS, SLT, SLP functions per IEC 62061

- Light curtain interface: Safety I/O terminals for perimeter guarding integration

- Collaborative safety monitor: Power and Force Limiting (PFL) mode via F/T sensor