BILL OF COMPONENTS (BOC)

MTR-S500

Machine Tending Robot (MTR-S500)

Version 1.0  |  2025

Industrial Robotics Division

# Component Register

The following table lists all major components by subsystem. Part numbers, supplier model references, and key parameters are provided for procurement and integration reference.

| Part ID | Name | Model/Supplier | Subsystem | Quantity | Key Parameters |
| --- | --- | --- | --- | --- | --- |
| ARM-01 | J1 Servo Motor | Yaskawa SGM7J-A5A | Actuator | 1 | 180Nm, resolver, brake |
| ARM-02 | J2 Servo Motor | Yaskawa SGM7J-A4A | Actuator | 1 | 160Nm, resolver, brake |
| ARM-03 | J3 Servo Motor | Yaskawa SGM7J-A3A | Actuator | 1 | 120Nm, resolver |
| ARM-04 | J4-J6 Servo Motors | Yaskawa SGM7J-A1A ×3 | Actuator | 1 | 20–40Nm, resolver |
| ARM-05 | J1-J3 Harmonic Drive | Harmonic Drive CSD-32 ×3 | Transmission | 1 | Ratio 100:1, <1 arc-min backlash |
| ARM-06 | J4-J6 Planetary Gearbox | Neugart PLE-60 ×3 | Transmission | 1 | Ratio 50:1, compact wrist |
| SEN-01 | 6-Axis F/T Sensor | ATI Mini45-SI-580-20 | Sensing | 1 | 300N/30Nm, 1kHz, EtherCAT |
| SEN-02 | 3D Structured-Light Camera | Photoneo PhoXi M | Sensing | 1 | 1.3MP, 0.1mm res, GigE |
| SEN-03 | 2D Barcode Scanner | Cognex DM300N | Sensing | 1 | QR/DataMatrix, LED ring |
| EEF-01 | Quick-Change Coupler | Schunk SWK-002 | End-Effector | 1 | Auto-lock, 300N, 50Nm |
| EEF-02 | Pneumatic Parallel Gripper | Schunk PGN-plus 80 | End-Effector | 1 | 80mm stroke, 300N |
| EEF-03 | Electric Adaptive Gripper | Robotiq 2F-140 | End-Effector | 1 | 140mm stroke, 3-pt |
| DRV-01 | Servo Drive J1-J3 | Yaskawa SGDV-R70A ×3 | Drive | 1 | EtherCAT, 5A cont |
| DRV-02 | Servo Drive J4-J6 | Yaskawa SGDV-R70A ×3 | Drive | 1 | EtherCAT, 2A cont |
| CPU-01 | Motion CPU | Beckhoff CX5140 | Compute | 1 | i7, 8GB RAM, TwinCAT RT |
| CPU-02 | Safety CPU | Pilz PSS 4000 | Safety | 1 | SIL3/PLe, dual-channel |
| CMM-01 | EtherCAT Master Hub | Beckhoff EK1100 | Comms | 1 | 8-port, 1ms cycle |
| CMM-02 | Profinet Gateway | Hilscher CIFX 50E-RE | Comms | 1 | Profinet IRT, 1ms |
| CMM-03 | OPC-UA Server Module | Beckhoff TF6100 | Comms | 1 | v1.04, VDMA 40010 |
| ELE-01 | Controller Cabinet | Rittal AX 1380.000 | Enclosure | 1 | 19-inch, 3U, IP54 |

# Procurement Notes

- All electronic components should be sourced from authorised distributors to ensure authenticity and warranty coverage.

- Safety-certified components (PLd/SIL2 or higher) must not be substituted without re-certification of the affected safety function.

- Minimum shelf life for LiFePO4 battery packs is 6 months from manufacture date at delivery; verify before acceptance.

- Harmonic drive reducers are matched to motor serial numbers at assembly; ensure correct pairing during spare parts replacement.