LIMITATIONS & CONSTRAINTS DOCUMENT

MTR-S500

Machine Tending Robot (MTR-S500)

Version 1.0  |  2025

Industrial Robotics Division

# 1. Introduction

This document describes the operational constraints, environmental restrictions, and technical limitations of the Machine Tending Robot (MTR-S500). Understanding these limitations is essential for safe deployment, system integration, and expectation management during commissioning.

# 2. Operational Limitations

## 2.1 Kinematics & Reach

- The 1350 mm reach is measured from the base flange centreline; the effective working envelope excludes a 200 mm dead zone around the base column.

- Simultaneous operation of all 6 axes near full extension at maximum speed generates joint torques that can trigger protective stops; trajectory planning must respect joint-speed coupling limits.

- The 7th linear axis option adds 40 ms latency to coordinated motion due to EtherCAT bus arbitration; not recommended for cycle times below 4 seconds.

## 2.2 Payload & Force

- 10 kg wrist payload is valid at ≤200 mm TCP offset; payloads at extended offsets require reduced speed profiles calculated via the built-in load estimation tool.

- The 6-axis F/T sensor rated at 300 N / 30 Nm will trigger safety-stop if threshold exceeded by >10%; overloads above 200% of rating cause permanent sensor drift.

- Pneumatic grippers require a stable 6-bar supply; fluctuations >±0.5 bar cause inconsistent clamping force and potential part drop.

## 2.3 Vision & Part Detection

- 3D structured-light camera performance degrades on highly specular surfaces (polished metal, glass); matte surface treatment or polarising filter required.

- Minimum part dimension detectable by the vision system is 8 mm; parts below this threshold require dedicated high-resolution camera module.

- Vision cycle time of 350 ms (3D scan + processing) sets a hard floor on takt time; faster cycles require GPU-accelerated inference hardware upgrade.

## 2.4 Safety & Collaboration

- ISO/TS 15066 collaborative mode limits TCP speed to 250 mm/s and reduces payload to 4 kg; full-speed operation requires a safety-rated guarding solution.

- The robot cannot self-detect collisions below 5 N without the optional F/T sensor; base configuration relies on motor current monitoring only.

- Emergency stop response time is 180 ms (software) to 40 ms (hardwired safety relay); hardwired E-Stop is mandatory for CE compliance.

# 3. Environmental Restrictions

| Parameter | Supported Range | Beyond Range Impact |
| --- | --- | --- |
| Temperature (operating) | 0°C to +45°C | Thermal shutdown, lubrication failure |
| Temperature (storage) | -20°C to +70°C | Seal/cable damage |
| Humidity (non-condensing) | 10% to 95% RH | Controller board corrosion |
| Vibration (base mounting) | ≤ 0.5 g at 5–55 Hz | Encoder noise, accuracy loss |
| Coolant exposure | IP65 arm rated | Controller not rated; must be shielded |
| Altitude | ≤ 2000 m ASL | Motor derating required above |
| EMI environment | Industrial Zone A | EtherCAT bus errors above Zone B |
| Floor levelness (base) | ≤ 0.1° tilt | Singularity in reachable workspace |

# 4. Integration Limitations

- EtherCAT master requires a dedicated real-time Linux kernel (PREEMPT-RT); Windows-based PLC controllers require Profinet gateway adapter.

- Native ROS2 driver supports Humble and Iron distributions only; Foxy and earlier are unsupported.

- OPC-UA server is read-only for telemetry; robot command via OPC-UA is not certified and requires a licensed middleware layer.

# 5. Maintenance-Related Constraints

- Joint grease intervals: J1–J3 every 5000 hours, J4–J6 every 3000 hours; missed intervals void positional accuracy warranty.

- The end-effector quick-change coupling must be inspected for wear every 10,000 change cycles; coupling torque below 85% of rated value requires replacement.

- The structured-light camera calibration drifts after thermal cycling; full recalibration is recommended after every 500 hours of camera-equipped operation or thermal shock events.