SYSTEM SCHEMATIC & SIGNAL FLOW

MTR-S500

Machine Tending Robot (MTR-S500)

Version 1.0  |  2025

Industrial Robotics Division

Note: This document presents textual/tabular schematic descriptions suitable for RAG model ingestion. Graphical CAD schematics are available as separate DXF/PDF artefacts.

# 1. Top-Level System Block Diagram

The MTR-S500 is organised into five primary subsystems: (1) Power & Electrical, (2) Motion Control, (3) Sensing, (4) End-Effector & Tooling, and (5) Communication. The safety CPU monitors all subsystems via a dedicated hardwired safety bus.

# 2. Power Flow Schematic

## 2.1 Power Distribution

| Source | Converter/Path | Load |
| --- | --- | --- |
| AC 3ø 480V | Servo Drive DC Bus | All 6 joint servo motors |
| AC 3ø 480V | SMPS 24VDC | Safety CPU, I/O modules |
| DC Bus 560V | Motor Driver J1 | J1 AC Servo Motor 180Nm |
| DC Bus 560V | Motor Driver J2 | J2 AC Servo Motor 160Nm |
| DC Bus 560V | Motor Driver J3–J6 | J3–J6 Servo Motors |
| 24V Bus | Direct | F/T Sensor, Encoders, I/O |
| 24V Bus | DC-DC 24→12V | Vision Camera, Laser Scanner |
| 24V Bus | Safety relay chain | E-Stop, light curtain, STO |

# 3. Signal & Data Flow Schematic

## 3.1 Internal Communication Bus

| Bus | Nodes Connected | Protocol / Rate |
| --- | --- | --- |
| EtherCAT (1 ms) | Motion CPU → J1–J6 Servo Drives | CoE CiA 402 |
| EtherCAT (1 ms) | Motion CPU → F/T Sensor | EtherCAT slave |
| EtherCAT (1 ms) | Motion CPU ← Encoder J1–J6 | Position feedback |
| Profinet (4 ms) | Robot Controller ↔ PLC | GSD device model |
| Ethernet (GbE) | Motion CPU ↔ Vision Camera | GigE Vision / ROS2 |
| USB 3.0 | Motion CPU ↔ Teach Pendant | HMI tether |
| GPIO 24V (safety bus) | Safety CPU ↔ E-Stop chain | Dual-channel IEC 62061 |
| GPIO 24V (safety bus) | Safety CPU → STO J1–J6 | Safe Torque Off |

# 4. External Interface Schematic

## 4.1 External Connections Summary

| M23 19-pin (power/safety) | Motor power + STO to arm from cabinet |
| --- | --- |
| M23 12-pin (encoder) | Resolver feedback cables J1–J3 |
| M12 8-pin × 3 (encoder) | Resolver feedback J4–J6 wrist axes |
| RJ45 (EtherCAT) | F/T sensor and optional external axes |
| GbE RJ45 (vision) | 3D camera GigE Vision connection |
| D-Sub 25 (safety I/O) | Light curtain, mat, area scanner inputs |
| Teach pendant 10-pin M16 | Pendant tether (power + USB 3.0 + E-Stop) |
| 3-phase CEE 63A plug | Mains power supply to controller cabinet |

# 5. Safety Architecture Schematic

The MTR-S500 safety architecture is certified to PLe / SIL 3. The safety CPU monitors joint velocity via the resolver feedback and compares against configured SLS/SOS limits at 1 kHz. On any limit breach, it simultaneously drops STO signals to all 6 servo drives via hardwired dual-channel I/O. The E-Stop chain is independent of the motion CPU; a single STO command takes 40 ms to achieve zero-torque state. Force-torque sensor output is monitored at 1 kHz by the safety CPU for PFL collaborative mode.