TECHNICAL SPECIFICATION

MTR-S500

Machine Tending Robot (MTR-S500)

Version 1.0  |  2025

Industrial Robotics Division

# 1. Scope & Purpose

This specification defines the functional, performance, mechanical, electrical, and software requirements for the Machine Tending Robot (MTR-S500). It serves as the authoritative reference for procurement, integration engineering, and acceptance testing.

# 2. Performance Specifications

## 2.1 Motion Performance

| Parameter | Specification | Test Method |
| --- | --- | --- |
| Rated payload (wrist) | 10 kg | ISO 9283 static load |
| Repeatability (RP) | ±0.02 mm | ISO 9283 §7.2 |
| Path accuracy (AP_T) | ≤ 0.5 mm | ISO 9283 §8.2 |
| Maximum TCP speed | 2000 mm/s | Linear move, no load |
| Maximum joint speed J1/J2/J3 | 180°/s | Single-axis move |
| Maximum joint speed J4/J5/J6 | 360°/s | Single-axis move |

## 2.2 Force/Torque Performance

| Parameter | Specification | Test Method |
| --- | --- | --- |
| F/T rated force (XYZ) | ±300 N | Static calibration rig |
| F/T rated torque (XYZ) | ±30 Nm | Static calibration rig |
| F/T resolution | 0.1 N / 0.01 Nm | Bench calibration |
| F/T sample rate | 1000 Hz | EtherCAT bus trace |
| Contact detection threshold | ≤ 5 N (with F/T sensor) | Controlled contact test |

## 2.3 Vision System Performance

| Parameter | Specification | Test Method |
| --- | --- | --- |
| 3D scan cycle time | ≤ 350 ms | Trigger-to-result timing |
| Point cloud resolution | 0.1 mm @ 0.5 m | Calibration plate test |
| Part detection accuracy | ±0.5 mm TCP correction | Known pose comparison |
| Minimum detectable part size | 8 mm | Part dimension gauge |
| Barcode reading distance | 100–800 mm | DataMatrix 10 mm cell |

# 3. Electrical Specifications

| Mains supply | 200–480V AC, 3-phase, 50/60 Hz |
| --- | --- |
| Peak power consumption | 4.8 kW |
| Rated power consumption | 2.5 kW |
| Inrush current (startup) | < 60 A (10 ms) |
| Cabinet supply fuse | 32 A (gG type) |
| 24V logic supply (external) | 10 A (user I/O) |
| Safety circuit voltage | 24V DC, dual-channel |
| EMC class | IEC 61000-6-4 (Class A industrial) |

# 4. Software & Interface Specifications

- Operating system: RT Linux (PREEMPT-RT kernel) + ROS2 Humble control layer

- Robot description: URDF/SRDF with MoveIt2 configuration package (open source)

- Motion planning: MoveIt2 with OMPL / TrajOpt solver; trajectory scaling API

- PLC interface: Profinet IO device (GSD file supplied), 10 ms cycle time

- OPC-UA: Server v1.04, Companion Spec VDMA 40010 (Robotics)

- REST API: OpenAPI 3.0 specification supplied; JWT bearer token auth

- Simulation: Gazebo Harmonic plugin; URDF compatible with Isaac Sim

- Teach pendant: Qt-based HMI, touch/keyboard, multi-language (EN, DE, ZH, JA)

# 5. Compliance & Standards

| ISO 10218-1:2011 | Robots — Safety requirements for industrial robots |
| --- | --- |
| ISO/TS 15066:2016 | Collaborative robot safety (PFL / SSM / HGP / SLS) |
| IEC 62061:2021 | Functional safety — SIL 3 safety functions |
| ISO 9283:1998 | Manipulating robots — performance criteria & test methods |
| CE (Machinery Directive) | 2006/42/EC full compliance |
| RIA R15.06 | North American robot safety standard |
| IEC 61800-5-2 | Drive systems — safety requirements |
| RoHS 3 (2015/863/EU) | Hazardous substances restriction |