USAGE, DEPLOYMENT & OPERATING GUIDE

MTR-S500

Machine Tending Robot (MTR-S500)

Version 1.0  |  2025

Industrial Robotics Division

# 1. Safety Precautions

Read this section in full before operating the system. Failure to observe safety requirements may result in personal injury, equipment damage, or production loss.

- Establish a safety-guarded work cell before enabling full-speed automatic operation.

- Never enter the robot's working envelope while the robot is in automatic or remote mode.

- Verify brake function by manually testing E-Stop before each production shift.

- Gripper must be confirmed closed (sensor feedback) before initiating any motion with a held part.

- Teach mode requires continuous deadman grip; releasing deadman grip immediately halts all motion.

# 2. Pre-Operation Checklist

- Inspect all joint axes for free movement; listen for unusual sounds.

- Verify all cables and hoses are routed correctly with no pinch points through the full working range.

- Check end-effector coupling lock indicator (green LED = locked).

- Confirm pneumatic supply pressure is 6.0 ±0.3 bar at the robot base manifold.

- Power on controller; verify no fault codes on pendant display.

- Run 'Auto Home' routine to verify joint reference positions match last saved values.

- Test E-Stop from pendant and external push button; confirm STO engages within 40 ms.

- Execute low-speed (10%) program trial run and verify all positions visually before running at full speed.

# 3. Deployment Procedure

## 3.1 Mechanical Installation

Mount the robot base on a level steel baseplate (flatness ≤ 0.1°) using the four M16 anchor bolts. Torque to 120 Nm. Route cable loom through the pedestal before bolting.

- Position base plate and mark anchor holes.

- Drill, sleeve, and torque anchor bolts.

- Lift robot arm using certified lifting ring (M12) onto base plate.

- Connect M23 power/encoder harness from controller cabinet.

- Route end-effector media (pneumatic, electric) through wrist cable guide.

- Power on and run 'Auto Home'; verify encoder reference positions.

## 3.2 Program Teaching & Setup

- Select programming mode on teach pendant.

- Jog robot to each key position (approach, pick, retract, place) at ≤ 10% speed.

- Record positions as waypoints in the job file.

- Define I/O triggers: gripper open/close, machine door signal handshake.

- Add force-controlled search motion for pick positions (±5 mm tolerance search spiral).

- Run full program at 25%, 50%, 75%, then 100% speed; verify cycle time and quality.

# 4. Normal Operation

During production, the robot operates in fully automatic mode with no operator interaction required:

- Monitor cycle time via the MES dashboard; deviations >5% warrant investigation.

- Respond to fault codes on the teach pendant; most require acknowledgement after root-cause resolution.

- Perform scheduled end-effector cleaning as required by the part material (e.g., swarf removal every 50 cycles).

- Verify part presence sensor and gripper feedback at start of each production batch.

- Log all alarm events in the maintenance book for trend analysis.

# 5. Maintenance Schedule

| Task | Interval | Personnel |
| --- | --- | --- |
| Joint lubrication J4–J6 | 3000 hours | Maintenance Tech |
| Joint lubrication J1–J3 | 5000 hours | Maintenance Tech |
| Brake test (all axes) | Monthly | Operator |
| End-effector coupling wear check | 10,000 change cycles | Maintenance Tech |
| Vision camera calibration | 500 hours | Process Engineer |
| Safety function test (STO, SLS) | 6 months | Safety Engineer |
| Resolver calibration verification | Annually | Commissioning Engineer |
| Servo drive firmware update | Annually | Commissioning Engineer |

# 6. Troubleshooting Quick Reference

| Symptom | Probable Cause | Recommended Action |
| --- | --- | --- |
| Position deviation fault on J2 | Brake drag or lubrication issue | Run brake test; apply lubricant per schedule |
| Gripper drops part mid-cycle | Pressure drop or sensor fault | Check pneumatic supply; verify sensor teach |
| Vision system fails to locate part | Lighting change or surface contamination | Clean part surface; re-run vision calibration |
| F/T sensor collision stop during search | Search spiral too aggressive | Reduce search force limit; increase approach speed |
| Cycle time increased by >10% | TCP speed reduced by fault history | Clear fault log; verify path is collision-free |
| EtherCAT bus error on J5 | Cable damage at wrist | Inspect cable routing through wrist |
| Program aborts at machine door open | I/O signal delay or configuration | Check PLC handshake timing; add dwell time |
| Safety stop on entry to collaborative zone | SLS threshold too low | Review TCP speed profile; re-teach boundary |

# 7. Decommissioning & Disposal

- Remove all programs and proprietary calibration data before disposal per data security policy.

- Servo drive capacitors retain dangerous charge for up to 5 minutes after power-off; wait before disassembly.

- Harmonic drive grease contains lithium soap; dispose as hazardous waste.

- Frame and motor cores may be recycled as industrial scrap metal.