# 01 – Mechanical System

The Mechanical System section covers all physical structural components of the Painting Robot, including load-bearing frames, articulated arm segments, end-effectors, and mechanical hardware such as joints, bearings, shafts, and fasteners.

## Subsections

| Folder | Description |
| --- | --- |
| Base_Frame/ | The rigid foundation that anchors the robot. Typically made from welded steel or aluminium profiles. Determines overall stability and mounting configuration |
| Upper_Arms/ | The proximal arm segments connected to the base. These carry the major kinematic loads and house larger servo/motor assemblies |
| Lower_Arms/ | The distal arm segments connecting to the end-effector. Designed for speed and lightweight construction to reduce inertia |
| End_Effector/ | The tool at the tip of the robot — a spray gun, brush, or roller mount. Determines the painting process type |
| Universal_Joints/ | Allow multi-axis rotation between arm segments. Critical for wrist flexibility and complex surface following |
| Bearings/ | Reduce friction at all rotating joints. Includes radial bearings, thrust bearings, and cross-roller bearings |
| Shafts/ | Transmit torque from motors to arm links. Designed for torsional rigidity and fatigue resistance |
| Fasteners/ | Bolts, nuts, pins, and retaining rings used throughout the assembly |

## Key Design Considerations

- Rigidity vs. Weight – Heavy frames improve stability but increase inertia; lightweight arms allow faster motion but may flex under load

- Reach & Workspace – Arm lengths and joint configurations define the reachable workspace

- Material Selection – Structural steel for base, aluminium alloy for arms, stainless steel for shafts in paint-exposed areas

- Repeatability – Mechanical play in bearings and joints directly affects painting accuracy

- Maintenance Access – Design allows for field replacement of bearings, end-effectors, and fasteners

## Related Sections

| Section | Relevance |
| --- | --- |
| 02_Motors | Motors mount to arm joints and drive shaft assemblies |
| 04_Sensors | Encoders and limit switches are embedded within the mechanical structure |
| 08_Kinematics | Arm geometry defines all kinematic calculations |
| 10_CAD | Full 3D models and drawings for every mechanical component |
| 13_Manufacturing | CNC, 3D printing, and assembly guides for fabricating parts |

# 02 – Motors

The Motors section covers all actuation hardware used to drive the robot's joints and auxiliary mechanisms. Three motor types are used depending on the torque, speed, and precision requirements of each joint.

## Subsections

| Folder | Description |
| --- | --- |
| Servo_Motors/ | High-precision closed-loop motors with integrated encoders. Used for joints requiring accurate position control |
| Stepper_Motors/ | Open-loop motors with discrete step resolution. Used for lower-precision, lower-speed auxiliary axes |
| BLDC_Motors/ | Brushless DC motors offering high speed and efficiency. Used in base rotation or high-speed linear axes |
| Gearboxes/ | Mechanical speed reducers (planetary, harmonic drive, spur gear) that increase torque output and reduce joint speed |

## Motor Type Comparison

| Property | Servo Motor | Stepper Motor | BLDC Motor |
| --- | --- | --- | --- |
| Control | Closed-loop | Open-loop | Closed/Open |
| Precision | Very High | High (step-based) | Medium-High |
| Torque | High | Medium | High |
| Speed | Medium | Low-Medium | Very High |
| Cost | High | Low | Medium |
| Typical Use | Main joints | Auxiliary axes | High-speed axes |

## Gearbox Types

| Type | Characteristics |
| --- | --- |
| Planetary | High torque density, compact, low backlash |
| Harmonic Drive | Near-zero backlash, ideal for precision wrist joints |
| Spur/Helical | Low cost, suitable for lower-precision applications |

## Related Sections

| Section | Relevance |
| --- | --- |
| 03_Motor_Drivers | Electronics that power and control these motors |
| 04_Sensors/Encoders | Feedback devices used with servo and BLDC motors |
| 06_Power_System | Power supply sizing based on motor current draw |
| 09_Control_Algorithms/PID | Closed-loop control tuning per motor |
| 15_Datasheets | Full motor and gearbox datasheets |

# 03 – Motor Drivers

The Motor Drivers section covers all drive electronics responsible for converting control signals from the controller into the power signals needed to run each motor. Drivers are the bridge between the control system and the physical actuators.

## Subsections

| Folder | Description |
| --- | --- |
| Servo_Drivers/ | Amplifiers and drives that receive position/velocity commands and power servo motors. Support modes: position, velocity, torque |
| Stepper_Drivers/ | Step-and-direction drivers that convert pulse signals to motor phase currents. Include microstepping support |
| Driver_Datasheets/ | Full manufacturer datasheets for all driver ICs and modules used in the project |

## Servo Drivers

Servo drives typically support:

- EtherCAT / CANopen / RS485 communication interfaces

- Position / Velocity / Torque control modes

- Auto-tuning of PID gains

- Fault detection: over-current, over-temperature, encoder error

- Regenerative braking for energy recovery during deceleration

## Stepper Drivers

Stepper drivers typically include:

- Microstepping resolution (1/2, 1/4, 1/8 ... 1/256 step)

- Current control (chopper-based) to limit heat dissipation

- Enable/disable pin for safety cut-off

- STEP / DIR / EN signal interface (TTL or differential)

Common ICs: A4988, DRV8825, TMC2209 (with UART tuning), TB6600

## Safety Considerations

- Always size drivers for 150-200% of motor rated current to allow acceleration headroom

- Use shielded cables between driver and motor to prevent EMI

- Implement hardware E-stop that disables driver enable lines

- Ensure proper heat sinking or forced-air cooling for continuous duty

## Related Sections

| Section | Relevance |
| --- | --- |
| 02_Motors | Motors powered by these drivers |
| 05_Control_System | Sends commands to drivers via CAN/RS485/EtherCAT |
| 06_Power_System | DC bus supply feeding all drivers |
| 07_Communication | CAN and RS485 wiring that connects drivers to the controller |
| 11_Electronics/Wiring_Diagrams | Physical wiring between drivers, motors, and PSU |

# 04 – Sensors

The Sensors section documents all feedback and perception hardware on the Painting Robot. Sensors enable closed-loop control, safety interlocking, environment awareness, and quality inspection.

## Subsections

| Folder | Description |
| --- | --- |
| Encoders/ | Rotary or linear position feedback devices mounted on motor shafts or joint axes |
| Limit_Switches/ | Mechanical or optical switches that detect joint travel boundaries and trigger homing routines |
| Proximity_Sensors/ | Non-contact sensors (inductive, capacitive, or optical) for object detection and collision avoidance |
| Vision_Systems/ | Cameras and image processing hardware for surface inspection, path correction, and colour matching |
| Force_Sensors/ | Load cells or torque sensors for contact-force feedback during brush/roller painting |

## Encoders

- Incremental encoders – Output pulse trains (A/B/Z channels); provide relative position and require homing

- Absolute encoders – Output digital position word (SSI, BiSS-C, EnDat); no homing required after power-on

- Resolution – Typically 1,000-65,536 PPR for incremental; 12-26-bit for absolute

- Interface – Differential RS422, SSI, or BiSS-C to controller/driver

## Limit Switches

- Used to define home position and hard travel limits on each axis

- Types: mechanical micro-switch, optical slot sensor, magnetic reed switch

- Normally-closed (NC) wiring recommended for fail-safe operation

## Proximity Sensors

- Inductive – Detect metallic objects (tool changes, fixture detection)

- Capacitive – Detect non-metallic materials (paint level, surface proximity)

- Optical / ToF – Distance measurement for surface standoff control during spraying

## Vision Systems

- 2D cameras – Surface defect detection, colour verification, path generation from image

- 3D depth cameras (structured light or stereo) – Surface mapping for complex geometry painting

- Interface – USB3, GigE Vision, or Camera Link to processing PC / Raspberry Pi

## Force / Torque Sensors

- Mounted at the wrist between the last link and end-effector

- Provide 6-axis force/torque data (Fx, Fy, Fz, Tx, Ty, Tz)

- Enable compliant motion – maintaining constant brush pressure against irregular surfaces

- Interface – RS485, CAN, or analog (0-10 V / 4-20 mA)

## Related Sections

| Section | Relevance |
| --- | --- |
| 02_Motors | Encoders are physically mounted on motor shafts |
| 05_Control_System | Controller reads all sensor inputs |
| 09_Control_Algorithms/Calibration | Sensor calibration procedures |
| 11_Electronics/Wiring_Diagrams | Sensor wiring to controller I/O |

# 05 – Control System

The Control System section covers all computing hardware and embedded platforms used to command the Painting Robot — from low-level microcontrollers running real-time loops to high-level single-board computers and PLCs.

## Subsections

| Folder | Description |
| --- | --- |
| STM32/ | ARM Cortex-M microcontrollers used for real-time motor control, PWM generation, and low-level I/O |
| Arduino/ | Arduino-based boards for prototyping, auxiliary I/O, and low-speed peripheral control |
| Raspberry_Pi/ | Linux SBC used for ROS/ROS2 nodes, vision processing, network communication, and HMI |
| PLC/ | Industrial Programmable Logic Controllers for production-grade reliability, safety interlocking, and ladder logic |
| Industrial_Controllers/ | Dedicated motion controllers (EtherCAT master, CNC controller) for multi-axis coordinated motion |

## Platform Roles

| Platform | Role |
| --- | --- |
| STM32 | Real-time control loop (1-10 kHz), encoder reading, PWM output, PID execution, CAN/UART communication |
| Arduino | Rapid prototyping and low-priority I/O: spray gun trigger, valve control, status LEDs |
| Raspberry Pi | Runs ROS2 middleware, executes MoveIt for motion planning, hosts HMI, interfaces with vision systems |
| PLC | Industrial-grade reliability (24/7), safety relay integration, E-stop logic, ladder/structured text programming |
| Industrial Controllers | EtherCAT master for synchronised multi-axis control; CNC for G-code based path execution |

## Real-Time Requirements

| Task | Cycle Time | Platform |
| --- | --- | --- |
| Current control loop | 20-100 µs | Motor driver DSP |
| Position/velocity PID | 0.5-2 ms | STM32 / Industrial controller |
| Trajectory interpolation | 1-10 ms | Motion controller |
| Path planning | 10-500 ms | Raspberry Pi / PC |
| Vision processing | 50-500 ms | Raspberry Pi / PC |

## Related Sections

| Section | Relevance |
| --- | --- |
| 03_Motor_Drivers | Receive commands from the control system |
| 04_Sensors | Feed data back into the control system |
| 07_Communication | Buses connecting all control nodes |
| 09_Control_Algorithms | Software running on these platforms |
| 12_Software | ROS2, MoveIt, and firmware code |

# 06 – Power System

The Power System section covers all hardware responsible for generating, converting, distributing, and protecting the electrical power supply of the Painting Robot.

## Subsections

| Folder | Description |
| --- | --- |
| SMPS/ | Switched-Mode Power Supplies that convert AC mains to regulated DC voltages for motors and electronics |
| Batteries/ | Battery packs for backup power, UPS functionality, or mobile/untethered robot configurations |
| Voltage_Regulators/ | DC-DC converters (buck, boost, LDO) for deriving multiple logic voltages from the main bus |
| Protection_Circuits/ | Fuses, circuit breakers, TVS diodes, crowbar circuits, and soft-start modules that safeguard all subsystems |

## Typical Voltage Rails

| Rail | Typical Voltage | Consumers |
| --- | --- | --- |
| Motor bus | 24V / 48V DC | Servo drivers, stepper drivers, BLDC drivers |
| Logic supply | 5V / 3.3V | Microcontrollers, sensors, encoders |
| SBC supply | 5V / 12V | Raspberry Pi, vision system |
| I/O supply | 24V DC | PLC I/O cards, relay coils, solenoid valves |
| Safety relay | 24V DC | E-stop loop, safety relay modules |

## Protection Circuits

| Device | Function |
| --- | --- |
| Fuse / circuit breaker | Overcurrent protection per branch |
| TVS diode | Transient voltage suppression on signal lines |
| Crowbar / clamp | Overvoltage protection on DC bus |
| Soft-start module | Limits inrush current at power-on |
| EMI filter | Reduces conducted interference from SMPS |
| Reverse polarity protection | P-channel MOSFET or Schottky diode at input |

## Related Sections

| Section | Relevance |
| --- | --- |
| 02_Motors | Primary power consumers |
| 03_Motor_Drivers | Powered by main DC bus |
| 05_Control_System | Logic and SBC power requirements |
| 11_Electronics/Schematics | Power distribution schematics |
| 11_Electronics/Wiring_Diagrams | Cable routing and wire gauges |

# 09 – Control Algorithms

The Control Algorithms section contains the software logic and mathematical methods that govern how the Painting Robot moves, tracks paths, and self-calibrates.

## Subsections

| Folder | Description |
| --- | --- |
| PID/ | Proportional-Integral-Derivative controllers for joint position, velocity, and force control |
| Motion_Control/ | Higher-level motion profiles, feedforward control, and multi-axis coordination |
| Path_Planning/ | Collision-free path generation from start to goal in joint or Cartesian space |
| Calibration/ | Procedures and algorithms for geometric, sensor, and force calibration |

## PID Control

PID is the foundation of closed-loop motor control. The output u(t) = Kp*e(t) + Ki*integral(e)dt + Kd*de/dt

| Term | Role |
| --- | --- |
| Kp (Proportional) | Reacts to current error — drives output toward setpoint |
| Ki (Integral) | Eliminates steady-state error — accumulates past errors |
| Kd (Derivative) | Predicts future error — dampens overshoot |

## Control Loop Hierarchy

Position loop (outer) -> Velocity loop (middle) -> Current loop (inner)

| Loop | Cycle Time | Platform |
| --- | --- | --- |
| Current loop | 10-50 kHz | Motor driver DSP |
| Velocity loop | 1-5 kHz | STM32 or driver |
| Position loop | 0.5-2 kHz | STM32 or motion controller |

## Motion Profiles

- Trapezoidal – Constant acceleration, cruise at max velocity, then decelerate. Most common profile

- S-Curve – Smoothed trapezoidal with jerk limiting, reduces mechanical shock. Recommended for high-precision painting

- Feedforward – Adds model-based anticipatory term to PID, reduces tracking error during continuous motion

## Path Planning

| Method | Description |
| --- | --- |
| Joint-space planning | Polynomial splines through via-points; no IK at intermediate points |
| Cartesian path planning | Define path as sequence of TCP poses; IK solved at each step |
| RRT / RRT* | Collision-free planning via OMPL library in MoveIt |
| Paint-specific | Spray overlap, standoff distance control, seam tracking |

## Calibration

- Geometric Calibration – Correct D-H parameter errors using least-squares fitting against measured poses

- Joint Zero Calibration (Homing) – Move to limit switch, record encoder count as home offset

- Hand-Eye Calibration – Determine camera-to-end-effector transform (Tsai-Lenz, Park-Martin methods)

- Force Sensor Calibration – Zero offset removal and gravity compensation based on tool weight

## Related Sections

| Section | Relevance |
| --- | --- |
| 04_Sensors | Provides feedback signals to all control loops |
| 05_Control_System | Hardware running these algorithms |
| 08_Kinematics | IK/FK used throughout motion and path planning |
| 12_Software/ROS2 | ROS2 nodes implementing these algorithms |
| 12_Software/MoveIt | Full pipeline from path planning to execution |

# 10 – CAD

The CAD section contains all 3D design files, assemblies, and drawings for the Painting Robot. These files are the authoritative source for geometry, tolerances, and manufacturing dimensions.

## Subsections

| Folder | Description |
| --- | --- |
| SolidWorks/ | Native SolidWorks (.SLDPRT, .SLDASM, .SLDDRW) part files, assemblies, and engineering drawings |
| Fusion360/ | Autodesk Fusion 360 cloud-native parametric models and integrated simulations |
| STEP_Files/ | Neutral STEP (.stp/.step) format exports for cross-software compatibility and vendor quoting |
| Assemblies/ | Top-level assembly files combining all subsystems into the complete robot model |

## File Type Reference

| Extension | Software | Description |
| --- | --- | --- |
| .SLDPRT | SolidWorks | Individual part file |
| .SLDASM | SolidWorks | Assembly file |
| .SLDDRW | SolidWorks | 2D engineering drawing |
| .f3d | Fusion 360 | Native parametric design |
| .step / .stp | Neutral | Industry-standard 3D exchange |
| .stl | Mesh | For 3D printing / simulation |
| .iges / .igs | Neutral | Legacy surface/solid exchange |

## Design Checks Before Manufacturing

- All mating faces have correct tolerances (H7/g6 for rotating shafts)

- Thread callouts match fasteners in BOM (11_Electronics/BOM)

- Weld symbols and surface finish specified on drawings

- Interference check run on full assembly — zero red collisions

- Mass properties reviewed — centre of gravity within stability envelope

- STEP file verified in neutral viewer before sending to CNC shop

## Simulation Capabilities

| Simulation Type | Tool | Purpose |
| --- | --- | --- |
| Static stress (FEA) | SW Simulation / Fusion Sim | Verify arm does not fail under load |
| Modal analysis | SW Simulation | Identify resonant frequencies to avoid in motion |
| Kinematic motion | SW Motion / Fusion Motion | Verify joint range of motion, interference |
| Thermal | SW Simulation | Motor and electronics heat dissipation |

## Related Sections

| Section | Relevance |
| --- | --- |
| 01_Mechanical_System | Physical components modelled in CAD |
| 08_Kinematics | D-H parameters extracted from CAD geometry |
| 13_Manufacturing | CAD files drive CNC programs and 3D print files |
| 11_Electronics/PCB_Designs | PCB enclosures modelled in CAD assemblies |
| 16_Metadata/components.csv | Part numbers link CAD files to the component registry |

# 11 – Electronics

The Electronics section contains all electrical design documentation: schematics, PCB layouts, bills of materials, and wiring diagrams for the Painting Robot's power, control, and sensor circuits.

## Subsections

| Folder | Description |
| --- | --- |
| Schematics/ | Circuit diagrams for all PCBs and sub-systems in PDF and native EDA format |
| PCB_Designs/ | Gerber files, drill files, and native PCB layout files (KiCad, Altium) |
| BOM/ | Bill of Materials — complete component lists with manufacturer part numbers, quantities, and suppliers |
| Wiring_Diagrams/ | Panel wiring diagrams, cable schedules, and connector pinout sheets |

## Key Schematic Sheets

| Sheet | Contents |
| --- | --- |
| Power Distribution | SMPS outputs, fusing, DC bus routing |
| Microcontroller Board | STM32 or Arduino schematic, decoupling, crystal |
| Motor Driver Interface | Driver enable/fault signals, current sense |
| Encoder Interface | Differential receiver, signal conditioning |
| I/O Expansion | Digital inputs (limit switches, E-stop), relay outputs |
| Communication Bus | CAN transceiver, RS485 transceiver, Ethernet PHY |

## PCB Design Rules

- Minimum trace width: 0.2 mm (signal), 2 mm (power >2A), 5 mm (motor bus)

- Ground planes on all layers under signal routing

- Differential pair routing for CAN, RS485, encoder signals (matched length ±0.5 mm)

- Creepage/clearance per IEC 60664-1 for 48V bus

- Test points on all critical nets

## BOM Columns

| Column | Description |
| --- | --- |
| Reference Designators | R1, C5, U3, J2 etc. |
| Description | Resistor 10kOhm 0402 1% |
| Manufacturer | Yageo, Murata, TI, etc. |
| MPN | Manufacturer part number |
| Quantity | Per board / system |
| Supplier | Digi-Key, Mouser, LCSC |
| Unit Price | For cost tracking |

## Wire Colour Code (IEC 60757)

| Colour | Function |
| --- | --- |
| Brown | L (live / phase) |
| Blue | N (neutral) |
| Green/Yellow | PE (protective earth) |
| Black | DC negative (0V) |
| Red | DC positive (+24V / +48V) |
| Orange | Control signal (24V logic) |
| Grey | Low-voltage signal (<5V) |

## Related Sections

| Section | Relevance |
| --- | --- |
| 03_Motor_Drivers | Driver wiring and enable/fault signal routing |
| 04_Sensors | Sensor connector pinouts and signal conditioning |
| 05_Control_System | Controller board schematics |
| 06_Power_System | Power distribution schematics |
| 07_Communication | Bus transceiver circuits |

# 12 – Software

The Software section contains all source code, firmware, simulation environments, and middleware configuration for the Painting Robot — from bare-metal microcontroller firmware to high-level ROS2 planning nodes.

## Subsections

| Folder | Description |
| --- | --- |
| ROS/ | ROS 1 (Noetic) packages for legacy hardware interface and tooling |
| ROS2/ | ROS 2 (Humble / Iron) packages — preferred for new development |
| MoveIt/ | MoveIt 2 configuration and plugins for motion planning, IK, and execution |
| Firmware/ | Embedded firmware for STM32, Arduino, and other microcontrollers |
| Simulations/ | Gazebo / RViz simulation worlds, URDF/XACRO robot models |

## Key ROS2 Packages

| Package | Function |
| --- | --- |
| painting_robot_description | URDF/XACRO robot model, collision/visual meshes |
| painting_robot_bringup | Launch files for full system startup |
| painting_robot_hw_interface | ros2_control hardware interface to drivers |
| painting_robot_moveit_config | MoveIt 2 config (SRDF, kinematics, controllers) |
| painting_robot_vision | Camera pipeline, surface mapping, path generation |
| painting_robot_teleop | Keyboard/joystick manual control |
| painting_robot_diagnostics | Health monitoring, logging, fault reporting |

## MoveIt 2 Configuration Files

| File | Purpose |
| --- | --- |
| robot.srdf | Semantic robot description (groups, disabled collisions) |
| kinematics.yaml | IK solver selection and parameters |
| ompl_planning.yaml | Planner selection and parameters |
| controllers.yaml | Joint trajectory controller mapping |
| joint_limits.yaml | Per-joint position, velocity, acceleration limits |

## Simulation Tools

| Tool | Purpose |
| --- | --- |
| Gazebo Classic / Harmonic | Physics simulation of robot and environment |
| RViz2 | Visualisation of robot state, planned paths, sensor data |
| MATLAB/Simulink | Control algorithm simulation before embedded deployment |
| CoppeliaSim (V-REP) | Rapid kinematics and sensor simulation |

## Software Development Practices

- Version control: Git (GitHub / GitLab), branching model: main -> develop -> feature/xxx

- CI/CD: GitHub Actions for automated build + unit tests on each pull request

- Coding standard: ROS2 style guide (ament_cpplint, ament_cmake_clang_format)

- Unit testing: ament_cmake_gtest for C++, pytest for Python nodes

- Documentation: Doxygen for C/C++ firmware; Sphinx for Python packages

## Related Sections

| Section | Relevance |
| --- | --- |
| 05_Control_System | Hardware that runs this software |
| 07_Communication | DDS / CAN / EtherCAT interfaces used by ROS2 nodes |
| 08_Kinematics | Mathematical models implemented in MoveIt / firmware |
| 09_Control_Algorithms | PID and motion algorithms implemented in firmware |
| 04_Sensors/Vision_Systems | Vision ROS2 nodes for camera integration |

# 13 – Manufacturing

The Manufacturing section covers all fabrication processes and assembly documentation needed to physically build the Painting Robot. It bridges CAD designs to physical parts.

## Subsections

| Folder | Description |
| --- | --- |
| CNC/ | G-code programs, CAM files, and fixturing documentation for CNC milled and turned parts |
| 3D_Printing/ | STL/3MF print files, slicer profiles, and print settings for prototype and functional parts |
| Laser_Cutting/ | DXF files and cut plans for sheet metal parts (brackets, panels, covers) |
| Assembly_Guides/ | Step-by-step assembly instructions, torque specifications, and illustrated work orders |

## CNC Machining — Design Rules

- Minimum inside corner radius = tool radius + 0.5 mm

- Avoid deep pockets with high aspect ratio (depth > 3x width)

- Add 0.1 mm machining stock to press-fit bores for finishing passes

- Specify surface finish (Ra 1.6 for bearing seats, Ra 3.2 general)

- Tolerances: H7/g6 for shaft-bearing fits; +-0.05 mm general

## 3D Printing Materials

| Material | Use Case |
| --- | --- |
| PLA | Prototyping, visual models |
| PETG | Functional parts (moderate temperature) |
| ABS | Impact-resistant housings |
| Nylon (PA12) | Snap-fit, flexible, wear parts |
| TPU | Vibration dampening mounts, cable grommets |
| PETG-CF / Nylon-CF | Structural brackets (high stiffness) |

## Laser Cutting — Material & Kerf

| Material | Thickness | Kerf (approx.) |
| --- | --- | --- |
| Mild steel | 1-3 mm | 0.1-0.2 mm |
| Stainless steel | 1-2 mm | 0.1-0.15 mm |
| Aluminium | 1-5 mm | 0.15-0.25 mm |
| Acrylic | 3-6 mm | 0.1-0.2 mm |

## Assembly Guide Structure

- Parts list — Reference BOM and CAD assembly

- Tools required — torque wrench, hex keys, bearing press

- Step-by-step procedure — numbered steps with exploded view images

- Torque specifications — per fastener size and class

- Critical checkpoints — inspection steps before proceeding

- Lubrication points — bearing grease type and quantity

- First-run checklist — pre-power checks after assembly

## Related Sections

| Section | Relevance |
| --- | --- |
| 10_CAD | Source CAD files for all manufacturing drawings |
| 01_Mechanical_System | Components fabricated by these processes |
| 11_Electronics/BOM | Parts lists referenced in assembly guides |
| 11_Electronics/PCB_Designs | Gerber files for PCB fabrication |
| 16_Metadata/components.csv | Component registry tracking sourcing and status |