01 — Mechanical System

Palletizing Robot | Structural & Mechanical Components

# Base Frame

The base frame is the fixed upper structure that houses all three servo motors and provides the rigid mounting point for the entire robot. It must withstand the combined torque and vibration loads of three high-speed actuators.

- Material: Aluminium alloy (6061-T6) or structural steel

- Machined motor flanges with precision bore for servo mount

- Triangular or circular plate geometry for symmetry

- Surface finishing: anodized or powder-coated for corrosion resistance

# Upper Arms

The three upper arms (also called proximal links) are rigidly connected to the output shaft of each servo motor. They transmit rotary motion from the motors into the spatial movement of the end-effector.

- Typical material: Carbon fibre rod or aluminium tube for low inertia

- Length determines workspace radius — commonly 200–400 mm

- Each arm connects to a lower arm assembly via a universal joint or ball joint

# Lower Arms (Parallelogram Links)

Each lower arm consists of two parallel rods (typically carbon fibre) connected at both ends via ball joints or rod-end bearings. This parallelogram geometry constrains the platform orientation.

- Dual-rod parallelogram per arm (6 rods total)

- Rod material: Carbon fibre or aluminium for stiffness/weight ratio

- End fittings: threaded rod-end bearings for length adjustment

- Critical for maintaining end-effector platform parallelism

# End Effector

The end-effector (or moving platform) is the output link where the tool (gripper, suction cup, etc.) is mounted. For palletizing, a vacuum gripper or mechanical gripper is typical.

- Lightweight aluminium or composite plate

- Mounts: vacuum cups, pneumatic gripper, or servo-driven fingers

- Optional 4th-axis rotary actuator for orientation control

# Joints, Bearings, Shafts & Fasteners

## Universal / Ball Joints

- Rod-end (Heim) joints for lower arm endpoints

- Universal joints for upper-to-lower arm transitions (some designs)

- Must allow multi-axis rotation with minimal backlash

## Bearings

- Deep groove ball bearings for motor shaft support

- Angular contact bearings where axial loads are significant

- Bearing preload must be managed for precision

## Shafts & Fasteners

- Motor output shafts: stainless or alloy steel, keyed or clamped

- M4–M8 socket head cap screws (ISO class 8.8 minimum)

- Thread-locking compound (Loctite 243) on critical joints