03 — Motor Drivers

Palletizing Robot | Servo Drivers, Stepper Drivers & Datasheets

# Motor Driver Overview

Motor drivers (also called motor drives or amplifiers) translate low-power control signals from the controller into the high-power waveforms needed to drive the motors. Each motor requires its own dedicated driver.

| Driver Type | Compatible Motor | Control Interface | Example Part |
| --- | --- | --- | --- |
| Servo Driver | AC/DC Servo | Pulse/Dir, CANopen, EtherCAT | Leadshine EL7-EC |
| Closed-Loop Stepper Driver | Stepper + Encoder | Pulse/Dir, UART | Leadshine HBS86H |
| BLDC FOC Driver | BLDC/PMSM | PWM, CAN, SPI | ODrive, VESC |

# Servo Drivers

Servo drivers receive position, velocity, or torque commands from the main controller and use closed-loop feedback from the motor encoder to precisely track the command. They handle current regulation, speed loops, and position loops internally.

- Three control modes: Position, Velocity, Torque

- Tunable PID gains for each loop

- Built-in protection: overcurrent, overvoltage, overtemperature

- Input voltage: 24–48 VDC (small) or 100–240 VAC (industrial)

- Communication: Pulse/Direction (step-dir), RS485 Modbus, CANopen, EtherCAT

- Configuration via USB/PC tuning software

# Stepper Drivers

Stepper drivers convert step/direction pulses from the controller into phased current waveforms for the stepper motor coils. Microstepping allows smooth movement by energizing coils at fractional steps.

- Chopper constant-current drive for efficient operation

- Configurable peak current (DIP switches or software)

- Microstepping: 1/1 to 1/256 depending on driver

- Common ICs: TMC2209, TMC5160, DRV8825, A4988

- Closed-loop variants add encoder input for stall detection

# Driver Datasheets

All driver datasheets should be stored in the Driver_Datasheets subfolder. Key parameters to document for each driver include:

- Supply voltage range and rated current

- Peak and continuous output current

- Control interface pinout and timing diagrams

- Alarm and fault output specifications

- Heat dissipation and thermal derating curves

- Parameter configuration register map (for serial/network drivers)