04 — Sensors

Palletizing Robot | Encoders, Limit Switches, Vision & Force Sensors

# Sensor Overview

Sensors are critical to safe, accurate robot operation. They provide feedback on position, velocity, environment, and forces to the control system.

| Sensor Type | Purpose | Interface |
| --- | --- | --- |
| Encoder | Motor position/velocity feedback | Quadrature, SSI, BiSS-C |
| Limit Switch | Home position & overtravel protection | Digital I/O |
| Proximity Sensor | Object detection on conveyor | PNP/NPN digital output |
| Vision System | Part identification & pose estimation | GigE, USB3, MIPI |
| Force Sensor | Grip force monitoring | SPI, I2C, Analog |

# Encoders

- Incremental quadrature encoders: 1000–5000 PPR typical

- Absolute encoders: single-turn (12–17 bit) or multi-turn

- Magnetic encoders (e.g. AS5047) for compact designs

- Optical encoders for highest resolution applications

- Output interfaces: RS422 differential (noise immunity), SSI, BiSS-C

# Limit Switches

- Mechanical micro-switches for hard homing

- Inductive proximity switches (IO-Link capable) for non-contact homing

- Normally-closed configuration recommended for fail-safe operation

- One switch per joint minimum; end-of-travel protection

# Proximity Sensors

- Inductive sensors: detect metal objects on conveyor

- Capacitive sensors: detect any material including cardboard/plastic

- Photoelectric sensors: beam-break for box detection

- Sensing range: 2–20 mm typical; adjustable sensitivity

# Vision Systems

Vision systems enable the robot to identify and locate objects without fixed positioning jigs, critical for unstructured palletizing environments.

- 2D cameras: barcode reading, color sorting, fiducial detection

- 3D depth cameras (Intel RealSense, ZED): 3D object localization

- Structured light scanners: high-accuracy surface scanning

- Software: OpenCV, Halcon, Cognex VisionPro, custom ROS nodes

# Force Sensors

- 6-axis force/torque sensors at the end-effector (e.g. ATI, OnRobot)

- Strain-gauge load cells for gripper force monitoring

- Enables compliant grasping and collision detection

- Important for fragile goods palletizing