05 — Control System

Palletizing Robot | STM32, Arduino, Raspberry Pi, PLC & Industrial Controllers

# Control Architecture Overview

The control system is the brain of the Delta robot, responsible for executing trajectories, reading sensors, managing I/O, and communicating with higher-level systems (SCADA, MES). A layered architecture is typical: high-level path planning on a PC/SBC, real-time motion control on an MCU or FPGA.

| Layer | Hardware | Responsibilities |
| --- | --- | --- |
| Supervisory | PC / Raspberry Pi / ROS | Path planning, HMI, communication |
| Motion Control | STM32 / DSP / FPGA | Trajectory interpolation, servo commanding |
| Drive Layer | Servo/Stepper Drivers | Current control, encoder processing |
| Field Layer | Sensors, I/O modules | Physical world interface |

# STM32 Microcontroller

- ARM Cortex-M4/M7 core — deterministic real-time execution

- Recommended: STM32F4, STM32H7 series for motor control

- Peripherals: advanced timers for PWM, SPI, UART, CAN

- HAL library or bare-metal for lowest latency

- Suitable for closed-loop joint control at 1–10 kHz

# Arduino

- Arduino Mega or Due for prototyping and learning phases

- AccelStepper library for stepper control

- Not recommended for production — non-deterministic OS

- Useful for peripheral control (LEDs, auxiliary sensors)

# Raspberry Pi

- Raspberry Pi 4/5 as single-board computer for ROS2 nodes

- Handles high-level task: path planning, vision processing, logging

- Not real-time without RT_PREEMPT kernel patch

- GPIO for direct sensor/actuator interfacing in simple setups

# PLC & Industrial Controllers

For production-grade palletizing robots, industrial PLCs and motion controllers are preferred for reliability, safety compliance (IEC 61508), and ease of integration with factory networks.

- Siemens S7-1500 / SIMOTION for high-end deployments

- Beckhoff TwinCAT: PC-based soft PLC with EtherCAT motion

- Allen-Bradley (Rockwell) ControlLogix with Kinetix servo drives

- IEC 61131-3 programming: Ladder, FBD, ST, SFC