06 — Power System

Palletizing Robot | SMPS, Batteries, Regulators & Protection Circuits

# Power System Overview

A reliable power system is essential for stable robot operation. The power architecture must cleanly separate high-power motor circuits from sensitive logic and sensor circuits, with appropriate filtering and protection.

| Rail | Voltage | Consumers | Source |
| --- | --- | --- | --- |
| Motor Power | 24–48 VDC | Servo/stepper drivers | SMPS |
| Logic Power | 5 VDC | MCU, sensors, comms | Voltage regulator |
| Control Power | 12 VDC (optional) | Relays, solenoids, fans | DC-DC converter |
| Emergency | Battery backup | Safety circuits | LiFePO4 / SLA |

# Switched Mode Power Supplies (SMPS)

- Input: 100–240 VAC; output: 24 VDC or 48 VDC

- Rated power: 200–600 W depending on motor count and payload

- Brands: Mean Well, Omron, Phoenix Contact

- Features: overvoltage protection, short-circuit protection, PFC

- Consider regenerative braking energy: add braking resistor or regen unit

# Batteries

- UPS battery backup for graceful shutdown on power loss

- LiFePO4: long cycle life, safe chemistry, flat discharge curve

- SLA (Sealed Lead Acid): low cost, heavy, suitable for emergency circuits

- BMS (Battery Management System) required for Li-based packs

# Voltage Regulators

- Linear regulators (LM7805, LDO): simple but inefficient for high currents

- DC-DC buck converters: efficient step-down (48V → 5V / 3.3V)

- Modules: LM2596, MP2307, or integrated modules from Murata/TDK

- Separate grounds and use LC filters to prevent motor noise propagation

# Protection Circuits

- Emergency Stop (E-Stop): hardware-hardwired to disable motor power

- Fuses: fast-blow fuses per driver for overcurrent protection

- TVS diodes and MOVs: transient voltage suppression on driver inputs

- Optoisolators: galvanic isolation between high-power and logic domains

- Reverse polarity protection: P-channel MOSFET or Schottky diode on DC inputs