09 — Control Algorithms

Palletizing Robot | PID, Motion Control, Path Planning & Calibration

# PID Control

PID (Proportional-Integral-Derivative) control is the foundational feedback control algorithm used in every servo loop. Three nested PID loops (current → velocity → position) are typically used in servo systems.

| Loop | Input | Output | Bandwidth |
| --- | --- | --- | --- |
| Current Loop | Current error | Motor voltage (PWM) | 1–10 kHz |
| Velocity Loop | Speed error | Current setpoint | 100–1000 Hz |
| Position Loop | Position error | Velocity setpoint | 10–100 Hz |

- Kp (Proportional): immediate response to error

- Ki (Integral): eliminates steady-state error

- Kd (Derivative): damps oscillation, improves response

- Anti-windup essential on integral term

- Tuning methods: Ziegler-Nichols, auto-tune, manual trial-and-error

# Motion Control

- Interpolation: generate intermediate waypoints between G-code or API targets

- Real-time scheduling: deterministic execution at fixed sample rate

- Feedforward: model-based motor torque prediction for faster response

- Decoupled control: treat each arm independently (valid for Delta geometry)

- Advanced: computed torque control, model predictive control (MPC)

# Path Planning

- Collision avoidance: bounding-box or swept-volume checks

- Palletizing pattern: row-column grid, herringbone, or optimized stacking

- Pick sequence optimization: nearest-neighbor or TSP heuristics

- Bin picking: integration with vision system for unordered workpieces

- Simulation-first: verify all paths in software before hardware execution

# Calibration

- Kinematic calibration: measure actual arm lengths, joint offsets

- Home position calibration: define reference zero for each joint

- Tool Centre Point (TCP) calibration: identify exact tool offset

- Gravity compensation: model arm weight for better low-speed accuracy

- Methods: laser tracker, coordinate measuring arm, vision-based calibration