11 — Electronics

Palletizing Robot | Schematics, PCB Design, BOM & Wiring Diagrams

# Electronics Overview

The electronics subsystem integrates all electrical components — MCUs, drivers, sensors, power distribution, and communication interfaces — into a coherent, manufacturable system. Good electronics design is critical for reliability in industrial environments.

# Schematics

- Capture tools: KiCad (open-source), Altium Designer, Eagle CAD

- Modular sheet hierarchy: power, MCU, drivers, I/O, communication

- Net naming conventions: VCC_5V, GND, MOTOR_PWM_1, ENC_A_1, etc.

- Symbol libraries: use manufacturer-provided or IPC-compliant symbols

- Schematic review: check for floating pins, missing decoupling caps, wrong net connections

# PCB Design

- 4-layer stackup: Signal, GND plane, PWR plane, Signal

- Motor drive traces: wide copper pours (≥2 mm for 5 A+), thermal reliefs

- Ground planes: keep analog and digital grounds separated; join at single point

- Decoupling: 100 nF + 10 μF ceramic caps at every IC power pin

- DRC checks: minimum trace width, annular ring, clearances

- Manufacturing: JLCPCB, PCBWay for prototyping; IPC Class 2 spec minimum

# Bill of Materials (BOM)

- BOM format: line items with MPN (manufacturer part number), qty, value, footprint, supplier

- Include approved alternates for critical components

- Export from CAD tool as CSV; sync with procurement system

- BOM includes: mechanical hardware + electronics + consumables

# Wiring Diagrams

- System wiring diagram: all cable connections between enclosure components

- Cable schedules: wire gauge, colour code, from/to terminal labels

- Connector types: JST-XH (signal), Phoenix Contact (power terminals), M12 (field sensors)

- Cable management: drag chains for moving arms, shielded cables for encoder signals

- Labelling: heat-shrink labels at both ends of every wire