12 — Software

Palletizing Robot | ROS, ROS2, MoveIt, Firmware & Simulations

# Software Architecture

The software stack covers everything from low-level embedded firmware running on microcontrollers up to high-level task planning and operator HMI. A modular, well-documented architecture enables parallel development and easier maintenance.

# ROS / ROS2

- ROS2 (Robot Operating System 2): recommended for new projects (ROS1 EOL 2025)

- Node-based architecture: perception, planning, control nodes communicate via topics/services/actions

- DDS middleware: real-time capable with QoS configuration

- Packages: ros2_control, nav2 (navigation), sensor drivers

- Language: Python (prototyping), C++ (production nodes)

# MoveIt 2

- Motion planning framework for ROS2

- URDF/SRDF robot model definition

- Planners: OMPL (probabilistic), CHOMP, Pilz Industrial Motion Planner

- Collision checking: FCL, OctoMap for environment

- GUI: RViz2 with MoveIt plugin for planning visualization

- API: move_group C++ and Python interface for programmatic control

# Firmware

- Target: STM32 with HAL or LL drivers; FreeRTOS for task scheduling

- Velocity/position control loops: run at 1–10 kHz in interrupt or RTOS task

- Communication: CANopen slave stack (CANopenNode library) or custom protocol

- Safety: watchdog timer, error states, E-Stop hardware interrupt

- Build system: STM32CubeIDE or CMake + arm-none-eabi-gcc

# Simulations

- Gazebo (ROS2 integration): physics simulation with sensor plugins

- MATLAB/Simulink: control algorithm development and verification

- Webots: lightweight robot simulator, good for kinematics testing

- CoppeliaSim (formerly V-REP): industry-grade simulation with Python API

- Digital twin: sync simulated model with real robot telemetry