13 — Manufacturing

Palletizing Robot | CNC, 3D Printing, Laser Cutting & Assembly Guides

# Manufacturing Overview

Manufacturing translates CAD designs into physical parts. A combination of CNC machining, 3D printing, and laser cutting is typically used for Delta robot prototyping. Production quantities may use casting or stamping.

# CNC Machining

- Used for: base plate, motor flanges, arm hubs, precision brackets

- Materials: aluminium 6061-T6, stainless steel 304, engineering plastics

- Tolerances: ±0.05 mm for bearing seats and shaft fits

- CAM software: Fusion 360 CAM, Mastercam, HSMWorks

- G-code post-processor must match the specific CNC machine controller

# 3D Printing

- Used for: non-structural prototypes, cable guides, jigs, enclosures

- Materials: PLA (prototyping), PETG (moderate load), ABS/ASA (heat), Nylon PA12 (functional)

- Process: FDM for large parts, SLA/MSLA for fine detail, MJF/SLS for production

- Design rules: avoid overhangs >45°, add clearance for press-fits (+0.2 mm)

- Infill: 40–100% for load-bearing parts

# Laser Cutting

- Used for: flat plates, brackets, enclosure panels, acrylic covers

- Materials: aluminium sheet (≤6 mm), mild steel, acrylic, plywood

- File format: DXF or SVG for laser path

- Kerf compensation: typically 0.1–0.2 mm depending on laser and material

- Bend allowance must be included in flat patterns for sheet metal folds

# Assembly Guides

- Step-by-step illustrated assembly procedures for each sub-assembly

- Torque specifications for all fasteners (e.g. M5: 4.5 Nm for 8.8 grade)

- Inspection checkpoints: alignment, runout, play measurement

- First article inspection (FAI) report template

- Tools required list: hex keys, torque wrench, dial indicator, bearing press