Cobot Key Limitations

1. Speed and Power Restrictions

Because cobots share workspaces with humans, their speed and force are actively restricted to prevent injuries.

- Throughput: They are unsuitable for applications requiring rapid cycle times or high-volume pick-and-place operations.

- Tooling Hazards: Even with slow cobot movements, adding sharp tools (like cutters or welding torches) nullifies their safety, requiring the use of standard industrial guarding.

2. Payload and Reach Constraints

Cobot arms are generally lightweight and flexible to ensure safety, which limits their physical capabilities.

- Weight limits: They are generally capped at payloads between 3 kg and 20 kg (with a few heavy-duty models reaching 35 kg). They cannot handle heavy-duty or bulk material handling.

- Reach: Workspace limits and vertical reach (e.g., stacking full pallets above 2.2 meters) are restricted compared to large industrial robots.

3. Precision and Rigidity

- Deflection under load: Because they are designed to be flexible (to "give" when contacting a human), the arm can slightly deviate from its path when carrying heavier loads.

- Not ideal for micro-precision: While easy to program via "hand-guiding," this method limits millimeter-level exactness required for high-precision trajectories.

4. Requirement for Human Supervision

Cobots are not designed to be completely autonomous for "lights-out" manufacturing. They rely heavily on human intuition and assistance, making them inefficient for facilities looking to automate completely unassisted.

5. Application and Integration Challenges

- Application Fit: Attempting to automate tasks with unpredictable variability (like highly random bin-picking) exceeds the performance envelope of a collaborative setup.

- Peripheral costs: To make cobots deployable in different scenarios, they require additional peripherals (like advanced vision systems or seam-tracking), which increase the overall initial investment.