ACTUATOR — MOTOR

Servo / BLDC Drive System

Delta Robot Robotics Knowledge Base

# 1. Component Overview

The delta robot uses three identical servo or BLDC motors mounted on the fixed base frame. Each motor drives one upper arm link. Precise, synchronised torque and position control across all three axes determines the end-effector's Cartesian position.

## 1.1 Metadata

| Name | BLDC / Servo Motor (3×) |
| --- | --- |
| Category | Actuator |
| Subcategory | Rotary Motor |
| Manufacturer (Example) | Maxon Motor AG |
| Model (Example) | EC-i 40 (BLDC 24V, 70W) |
| Document Type | Datasheet / Specification |
| Source URL | https://www.maxongroup.com |
| Date Collected | 2024-06-01 |
| Collected By | Team Member — Actuator Lead |
| File Name | Motor_Maxon_ECi40_Datasheet.pdf |

# 2. Electrical Specifications

| Parameter | Value | Unit | Notes |
| --- | --- | --- | --- |
| Nominal Voltage | 24 | V DC | 3-phase BLDC |
| Continuous Current | 3.17 | A | At rated torque |
| Peak Current | 9.5 | A | Short duration |
| Winding Resistance (phase-phase) | 0.94 | Ω | At 25 °C |
| Winding Inductance (phase-phase) | 0.22 | mH |  |
| Back-EMF Constant | 18.4 | mV/rpm |  |
| Motor Constant | 25.8 | mNm/√W |  |
| Torque Constant | 56.9 | mNm/A |  |
| Thermal Resistance (winding-ambient) | 6.3 | K/W |  |

# 3. Mechanical Specifications

| Parameter | Value | Unit |
| --- | --- | --- |
| Rated Torque (continuous) | 180 | mNm |
| Peak Torque | 540 | mNm |
| No-Load Speed | 8000 | rpm |
| Rated Speed | 7430 | rpm |
| Rotor Inertia | 69.1 | g·cm² |
| Mass | 320 | g |
| Shaft Diameter | 8 | mm |
| Flange Diameter | 40 | mm |
| Overall Length | 71.2 | mm |

# 4. Encoder / Feedback

Integrated Encoder Details

| Parameter | Value |
| --- | --- |
| Encoder Type | Incremental Magnetic (Hall + Encoder) |
| Resolution | 1024 counts/rev (4096 with quadrature) |
| Output | A, B, Index (TTL) |
| Supply Voltage | 5 V |
| Max Speed | 12 000 rpm |
| Connector | 6-pin JST-ZH |

# 5. Wiring / Pinout

## 5.1 Motor Power Connector (3-phase)

| Pin | Label | Description |
| --- | --- | --- |
| 1 | U | Phase U (motor winding) |
| 2 | V | Phase V (motor winding) |
| 3 | W | Phase W (motor winding) |
| PE | GND Shield | Chassis / shield ground |

## 5.2 Encoder Connector (6-pin JST-ZH)

| Pin | Signal | Voltage | Description |
| --- | --- | --- | --- |
| 1 | VCC | 5 V | Encoder supply |
| 2 | GND | 0 V | Ground |
| 3 | A+ | 5 V TTL | Channel A |
| 4 | A− | 5 V TTL | Channel A inverted |
| 5 | B+ | 5 V TTL | Channel B |
| 6 | Index | 5 V TTL | One pulse per revolution |

# 6. Required Documents Checklist

| Document | Priority | Status | File Name |
| --- | --- | --- | --- |
| Datasheet | Required | Collect | Motor_Maxon_ECi40_Datasheet.pdf |
| Product Specifications | Required | Collect | Motor_Maxon_ECi40_Specs.pdf |
| CAD / STEP File | Required | Collect | Motor_Maxon_ECi40_CAD.step |
| Application Notes | Preferred | Collect | Motor_Maxon_ECi40_AppNote.pdf |
| Wiring Diagram | Preferred | Collect | Motor_Maxon_ECi40_Wiring.pdf |
| Encoder Manual | Preferred | Collect | Motor_Maxon_ECi40_Encoder.pdf |
| PCB Schematic | Advanced | Collect | Motor_Maxon_ECi40_PCB.pdf |
| Example Code (FOC) | Advanced | Collect | Motor_Maxon_ECi40_Code.zip |

# 7. Application Notes

## 7.1 Mounting

- Align motor shaft axis precisely with delta arm pivot axis to prevent side-loading.

- Use a rigid motor bracket; flex causes oscillation at high acceleration.

- Torque motor fixing bolts to manufacturer spec — typically M3 at 0.5 N·m.

## 7.2 Thermal Management

- Mount motors with heatsink contact surface facing aluminium frame.

- Use thermal paste between motor flange and mount when ambient > 35 °C.

- Monitor winding temperature via driver; derate above 100 °C.

## 7.3 Driver Compatibility

- Use FOC (Field-Oriented Control) servo driver for best torque linearity.

- Set current loop bandwidth ≥ 2 kHz for responsive position control.

- Match driver bus voltage to motor nominal voltage ± 10 %.

# 8. Daily Workflow Reference

- Find / source motor from approved vendor list.

- Download official datasheet from manufacturer website (not third-party).

- Rename: Motor_[Manufacturer]_[Model]_Datasheet.pdf

- Upload to shared drive under /Components/Actuators/Motors/

- Add Notion entry with all required metadata fields.

- Mark task completed in project tracker.