END EFFECTOR

Gripper / Tool Interface

Delta Robot Robotics Knowledge Base

# 1. Component Overview

The end effector is the moving platform payload interface of the delta robot. It connects to the three lower arm rods via ball joints and carries the tool — typically a vacuum suction cup, pneumatic gripper, or tool-changer. The moving platform mass directly impacts dynamic performance and must be minimised.

## 1.1 Metadata

| Name | Vacuum Suction End Effector |
| --- | --- |
| Category | End Effector |
| Subcategory | Vacuum Gripper |
| Manufacturer (Example) | Schmalz GmbH |
| Model (Example) | FSGPL 20 ED-55 G1/8-IG |
| Document Type | Datasheet / Specification |
| Source URL | https://www.schmalz.com |
| Date Collected | 2024-06-01 |
| Collected By | Team Member — End Effector Lead |
| File Name | EndEffector_Schmalz_FSGPL20_Datasheet.pdf |

# 2. Mechanical Specifications

| Parameter | Value | Unit | Notes |
| --- | --- | --- | --- |
| Cup Diameter | 20 | mm | Nominal |
| Stroke (compressed) | 8 | mm | Spring pre-compressed |
| Max Lifting Force (dry) | 2.8 | N | At −0.7 bar vacuum |
| Operating Temperature | −10 to +60 | °C |  |
| Moving Platform Mass | < 150 | g | Platform + tool combined target |
| Ball Joint Thread | M6 | — | Lower arm rod interface |
| Material (cup) | NBR / Silicone | — | Choose by payload surface |
| IP Rating | IP54 | — | Dust/splash protected |

# 3. Pneumatic Specifications

| Parameter | Value | Unit |
| --- | --- | --- |
| Operating Vacuum | −0.3 to −0.9 | bar |
| Port Thread | G1/8 | BSP |
| Max Flow Rate | 40 | NL/min |
| Vacuum Generator Type | Ejector (venturi) or pump | — |
| Solenoid Valve Voltage | 24 V DC | — |
| Response Time (ON/OFF) | < 8 | ms |

# 4. Electrical / Solenoid Valve Pinout

| Pin | Signal | Voltage | Description |
| --- | --- | --- | --- |
| 1 | VCC | 24 V DC | Solenoid power |
| 2 | GND | 0 V | Common ground |
| 3 | SENSE | 24 V logic | Vacuum sensor output (optional) |

# 5. Required Documents Checklist

| Document | Priority | Status | File Name |
| --- | --- | --- | --- |
| Datasheet | Required | Collect | EndEffector_Schmalz_FSGPL20_Datasheet.pdf |
| Product Specifications | Required | Collect | EndEffector_Schmalz_FSGPL20_Specs.pdf |
| CAD / STEP File | Required | Collect | EndEffector_Schmalz_FSGPL20_CAD.step |
| Wiring Diagram | Preferred | Collect | EndEffector_Schmalz_FSGPL20_Wiring.pdf |
| Pneumatic Schematic | Preferred | Collect | EndEffector_Schmalz_FSGPL20_Pneumatic.pdf |
| Application Notes | Preferred | Collect | EndEffector_Schmalz_FSGPL20_AppNote.pdf |

# 6. Design & Integration Notes

## 6.1 Platform Mass Budget

- Target total moving platform mass < 150 g for high-speed (> 5 m/s) pick-and-place.

- Use carbon-fibre or aluminium 5052 for platform plate; avoid steel.

- Route pneumatic tube through one lower arm to reduce external cable drag.

## 6.2 Ball Joint Selection

- Use IGUS KBRM-02 or equivalent low-friction rod-end ball joints.

- Minimum ball joint angular range ±30° for typical delta workspace.

- Pre-load ball joints to remove backlash; do not over-tighten (causes stiction).

## 6.3 Vacuum Circuit

- Install a small buffer volume (10 ml) near end effector to maintain vacuum during rapid motion.

- Use M5 push-in fittings and 4 mm OD tube rated for ≥ 10 bar.

- Fit inline filter (40 µm) to protect vacuum generator from debris.