MOTION CONTROLLER

Microcontroller / FPGA / Industrial PC

Delta Robot Robotics Knowledge Base

# 1. Component Overview

The motion controller executes the delta robot inverse kinematics, trajectory planning, and real-time servo loop closure. Typical implementations use an ARM Cortex-M7 microcontroller (e.g. STM32H7), an FPGA-based motion card, or an industrial PC with EtherCAT master running at 1 kHz or higher.

## 1.1 Metadata

| Name | STM32H743ZIT6 Microcontroller |
| --- | --- |
| Category | Controller |
| Subcategory | Microcontroller (MCU) |
| Manufacturer | STMicroelectronics |
| Model | STM32H743ZIT6 |
| Document Type | Datasheet / Reference Manual |
| Source URL | https://www.st.com/en/microcontrollers-microprocessors/stm32h743zi.html |
| Date Collected | 2024-06-01 |
| Collected By | Team Member — Controls Lead |
| File Name | Controller_STM_STM32H743_Datasheet.pdf |

# 2. Core Specifications

| Parameter | Value | Notes |
| --- | --- | --- |
| CPU Core | ARM Cortex-M7 | Single core |
| Clock Speed | 480 MHz | Max (with PLL) |
| Flash Memory | 2 MB | Internal |
| SRAM | 1 MB | Including TCM |
| FPU | Double-precision | IEEE 754 |
| DSP Instructions | Yes | SIMD |
| Operating Voltage | 1.71 – 3.6 V | Core 1.2 V (internal LDO) |
| Operating Temperature | −40 to +85 °C | Industrial grade |
| Package | LQFP-144 | 14×14 mm |

# 3. Peripherals Relevant to Delta Robot

| Peripheral | Count | Use in Delta Robot |
| --- | --- | --- |
| Advanced Timer (TIM1/TIM8) | 2 | 3-phase PWM generation for 3 motor drivers |
| General Timer | 12 | Encoder counting (QEI mode) per axis |
| SPI | 6 | High-speed sensor / IMU communication |
| I2C | 4 | Peripheral configuration |
| UART / USART | 8 | Debug, HMI, secondary comms |
| CAN-FD | 2 | Industrial fieldbus / multi-axis sync |
| Ethernet MAC | 1 | EtherCAT / Modbus TCP (with PHY IC) |
| ADC (16-bit) | 3 | Current sensing, analogue force sensors |
| DAC (12-bit) | 2 | Analogue velocity reference (legacy drivers) |
| DMA Channels | 16+ | Zero-CPU peripheral data transfer |

# 4. Key Pinout Groups

## 4.1 Motor PWM Output (TIM1 — 3-phase complementary)

| Pin | Function | Motor Axis |
| --- | --- | --- |
| PE9 | TIM1_CH1 | Axis 1 — High-side U |
| PE8 | TIM1_CH1N | Axis 1 — Low-side U |
| PE11 | TIM1_CH2 | Axis 1 — High-side V |
| PE10 | TIM1_CH2N | Axis 1 — Low-side V |
| PE13 | TIM1_CH3 | Axis 1 — High-side W |
| PE12 | TIM1_CH3N | Axis 1 — Low-side W |

## 4.2 Encoder Inputs (QEI Mode)

| Timer | Pins (A, B) | Axis |
| --- | --- | --- |
| TIM2 (32-bit) | PA0, PA1 | Axis 1 encoder |
| TIM3 (32-bit) | PC6, PC7 | Axis 2 encoder |
| TIM4 (32-bit) | PD12, PD13 | Axis 3 encoder |

# 5. Software Stack

## 5.1 Real-Time OS

- FreeRTOS v10.x — recommended for servo loop + communications tasks.

- Servo loop task priority: highest (preempts all others).

- Loop period: 500 µs (2 kHz) minimum for smooth BLDC control.

## 5.2 Key Libraries

| Library / Framework | Purpose |
| --- | --- |
| STM32 HAL / LL | Hardware abstraction and low-level register access |
| CMSIS-DSP | PID, FFT, matrix ops for kinematics |
| lwIP | Lightweight TCP/IP stack (EtherCAT / Modbus TCP) |
| FreeRTOS | Real-time scheduling |
| Custom Delta IK | Inverse kinematics library (team-developed) |

# 6. Required Documents Checklist

| Document | Priority | Status | File Name |
| --- | --- | --- | --- |
| Datasheet | Required | Collect | Controller_STM_STM32H743_Datasheet.pdf |
| Reference Manual (RM0433) | Required | Collect | Controller_STM_STM32H743_RefManual.pdf |
| Programming Manual | Required | Collect | Controller_STM_STM32H743_ProgManual.pdf |
| CAD Footprint | Required | Collect | Controller_STM_STM32H743_CAD.step |
| Application Notes (AN4013) | Preferred | Collect | Controller_STM_STM32H743_AN4013.pdf |
| Eval Board Schematic (NUCLEO-H743) | Preferred | Collect | Controller_STM_NUCLEOH743_Schematic.pdf |
| Example Code (motor control) | Advanced | Collect | Controller_STM_STM32H743_MotorCtrl.zip |