POWER SUPPLY

24 V DC Industrial PSU & Distribution

Delta Robot Robotics Knowledge Base

# 1. Component Overview

A regulated 24 V DC power supply provides energy to the motor drivers, controller, solenoid valves, and sensors. The system uses a primary 24 V bus for high-power loads and a secondary 5 V / 3.3 V rail (via DC-DC converters) for logic. Proper protection and distribution is critical for safety and uptime.

## 1.1 Metadata

| Name | 24 V / 20 A DIN Rail PSU |
| --- | --- |
| Category | Power |
| Subcategory | DC Power Supply |
| Manufacturer (Example) | MEAN WELL |
| Model (Example) | SDR-480P-24 |
| Document Type | Datasheet / Specification |
| Source URL | https://www.meanwell.com |
| Date Collected | 2024-06-01 |
| Collected By | Team Member — Electrical Lead |
| File Name | Power_MeanWell_SDR480P24_Datasheet.pdf |

# 2. Electrical Specifications

| Parameter | Value | Unit | Notes |
| --- | --- | --- | --- |
| Input Voltage Range | 90 – 264 | V AC | Universal input |
| Input Frequency | 47 – 63 | Hz |  |
| Output Voltage | 24 | V DC | Adjustable ±15% |
| Output Current (rated) | 20 | A | 480 W total |
| Output Ripple & Noise | < 150 | mV p-p | At full load |
| Regulation (load) | < 1 | % |  |
| Efficiency | ≥ 91 | % | At full load 230 V |
| Hold-up Time | 20 | ms | At full load 115 V |
| Inrush Current | < 60 | A | Cold start, 230 V |
| Operating Temperature | −20 to +70 | °C | Derate above 50°C |

# 3. Protection Features

- Over-Voltage Protection (OVP): 27.6–32.4 V — auto-recovery or latch.

- Over-Current Protection (OCP): 105–135% of rated current — hiccup mode.

- Over-Temperature Protection (OTP): shuts down; recovers when cool.

- Short-Circuit Protection (SCP): hiccup mode, auto-recover.

- Isolation: 3000 V AC input-output, 1500 V AC output-ground.

# 4. Power Distribution Architecture

Power Rail Overview

| Rail | Voltage | Source | Consumers |
| --- | --- | --- | --- |
| Main Bus | 24 V DC | Primary PSU | Motor drivers (3×), solenoid valves |
| Logic Rail | 5 V DC | DC-DC (LM2596) | Encoders, fans, logic I/O |
| MCU Rail | 3.3 V DC | LDO from 5V (AMS1117) | STM32, IMU, SPI devices |
| Emergency Stop | 24 V DC (switched) | Safety relay | All motor drivers enable pins |

# 5. Wiring & Connector Pinout

## 5.1 PSU Output Terminals

| Terminal | Label | Description |
| --- | --- | --- |
| L+ | +24 V | Positive output rail |
| L− | 0 V | Negative / common ground |
| PE | ⏚ | Protective earth (chassis) |
| -S / +S | Remote sense | Optional: compensate cable drop |
| CNT | Control | Remote ON/OFF (short to L+ = ON) |

# 6. Required Documents Checklist

| Document | Priority | Status | File Name |
| --- | --- | --- | --- |
| Datasheet | Required | Collect | Power_MeanWell_SDR480P24_Datasheet.pdf |
| Product Specifications | Required | Collect | Power_MeanWell_SDR480P24_Specs.pdf |
| CAD / DIN-Rail Dimensions | Required | Collect | Power_MeanWell_SDR480P24_CAD.step |
| Wiring Diagram | Preferred | Collect | Power_MeanWell_SDR480P24_Wiring.pdf |
| Derating Curves | Preferred | Collect | Power_MeanWell_SDR480P24_Derating.pdf |
| Safety / Compliance Cert | Advanced | Collect | Power_MeanWell_SDR480P24_Certs.pdf |

# 7. Safety Notes

- Always fuse each output branch — use blade fuses rated at 125% of branch current.

- Ensure chassis earth (PE) is bonded to machine frame at a single point.

- Install E-stop safety relay in series with all motor driver ENABLE lines.

- Use shielded cable for encoder signals; ground shield at controller end only.

- Label all cables with voltage and circuit ID in accordance with IEC 60204-1.