SENSORS

IMU · Encoder · Force / Torque

Delta Robot Robotics Knowledge Base

# 1. Component Overview

The delta robot uses three classes of sensors: (1) rotary encoders for motor shaft position feedback, (2) an inertial measurement unit (IMU) for platform orientation and vibration monitoring, and (3) optional force/torque sensors at the tool interface for contact detection and compliant assembly tasks.

# 2. IMU — Inertial Measurement Unit

## 2.1 Metadata

| Name | BNO055 |
| --- | --- |
| Category | Sensor |
| Subcategory | IMU (9-DOF) |
| Manufacturer | Bosch Sensortec |
| Model | BNO055 |
| Document Type | Datasheet |
| Source URL | https://www.bosch-sensortec.com/products/smart-sensor-systems/bno055/ |
| Date Collected | 2024-06-01 |
| Collected By | Team Member — Sensor Lead |
| File Name | Sensor_Bosch_BNO055_Datasheet.pdf |

## 2.2 Electrical Specifications

| Parameter | Value | Unit |
| --- | --- | --- |
| Supply Voltage (VDD) | 2.4 – 3.6 | V |
| Supply Voltage (VDDIO) | 1.7 – 3.6 | V |
| Current (fusion mode) | 4.9 | mA |
| Current (suspend) | 2.0 | µA |
| Communication Interfaces | I2C (400 kHz) / SPI (1 MHz) | — |
| I2C Address (default) | 0x28 (ADR pin LOW) | — |

## 2.3 Sensor Performance

| Axis | Range | Resolution | Noise |
| --- | --- | --- | --- |
| Accelerometer | ±2 / ±4 / ±8 / ±16 g | 1 mg | 150 µg/√Hz |
| Gyroscope | ±125 to ±2000 °/s | 0.061 °/s | 0.014 °/s/√Hz |
| Magnetometer | ±1300 µT (XY), ±2500 µT (Z) | 0.3 µT | — |
| Fusion Output | Absolute orientation | 0.1° | ±2.5° heading |

## 2.4 Pinout (LGA-28)

| Pin | Label | Description |
| --- | --- | --- |
| 1 | VDD | Main supply (2.4–3.6 V) |
| 2 | VDDIO | I/O supply |
| 3 | GND | Ground |
| 4 | SDA / SDI | I2C data / SPI MOSI |
| 5 | SCL / SCLK | I2C clock / SPI clock |
| 6 | PS0 / WAKE | Protocol select (LOW = I2C) |
| 7 | INT | Interrupt output (active high) |
| 8 | NRST | Reset (active low) |

# 3. Rotary Encoder

## 3.1 Metadata

| Name | AMT102-V Incremental Encoder |
| --- | --- |
| Category | Sensor |
| Subcategory | Rotary Encoder |
| Manufacturer | CUI Devices |
| Model | AMT102-V |
| Source URL | https://www.cuidevices.com/product/motion/rotary-encoders/incremental/modular/amt10-series |
| File Name | Sensor_CUI_AMT102V_Datasheet.pdf |

## 3.2 Specifications

| Parameter | Value | Unit |
| --- | --- | --- |
| Resolution (programmable) | 48 – 2048 | PPR |
| Supply Voltage | 3.3 – 5.0 | V |
| Output Type | Push-pull (A, B, Index) | — |
| Max Speed | 8000 | RPM |
| Shaft Diameter | 6 (or 8 via adapter) | mm |
| Operating Temperature | −40 to +125 | °C |

# 4. Force / Torque Sensor (Optional)

## 4.1 Metadata

| Name | ATI Mini45 F/T Sensor |
| --- | --- |
| Category | Sensor |
| Subcategory | Force / Torque |
| Manufacturer | ATI Industrial Automation |
| Model | Mini45 (SI-160-4) |
| Source URL | https://www.ati-ia.com/products/ft/ft_models.aspx?id=Mini45 |
| File Name | Sensor_ATI_Mini45_Datasheet.pdf |

## 4.2 Specifications

| Parameter | Value |
| --- | --- |
| Force Range (Fx, Fy) | ±160 N |
| Force Range (Fz) | ±240 N |
| Torque Range (Tx, Ty, Tz) | ±4 N·m |
| Resolution (force) | 1/40 N |
| Interface | SPI / RS-422 / Ethernet (option) |
| Sampling Rate | 7000 Hz |
| Operating Temperature | 0 – 50 °C |

# 5. Required Documents Checklist

| Component | Document | Priority | File Name |
| --- | --- | --- | --- |
| BNO055 IMU | Datasheet | Required | Sensor_Bosch_BNO055_Datasheet.pdf |
| BNO055 IMU | Application Note (BST-BNO055-AN007) | Preferred | Sensor_Bosch_BNO055_AppNote.pdf |
| AMT102-V Encoder | Datasheet | Required | Sensor_CUI_AMT102V_Datasheet.pdf |
| AMT102-V Encoder | CAD Model | Required | Sensor_CUI_AMT102V_CAD.step |
| ATI Mini45 | Datasheet | Required | Sensor_ATI_Mini45_Datasheet.pdf |
| ATI Mini45 | Communication Manual | Required | Sensor_ATI_Mini45_CommManual.pdf |
| ATI Mini45 | CAD / STEP | Preferred | Sensor_ATI_Mini45_CAD.step |