STRUCTURAL / MECHANICAL

Base Frame · Upper Arms · Lower Rods · Moving Platform

Delta Robot Robotics Knowledge Base

# 1. Component Overview

The delta robot mechanical structure consists of a fixed base frame, three upper arms (driven by motors), six lower rod links (arranged in three pairs), and a moving platform. All joints use precision ball joints. The geometry determines the workspace volume, accuracy, and dynamic capability.

## 1.1 Metadata

| System | Delta Robot Mechanical Structure |
| --- | --- |
| Category | Mechanical |
| Subcategory | Robot Frame |
| Material (frame) | Aluminium 6061-T6 / Carbon Fibre Tube |
| Design Standard | Custom (IEC 60068 environmental compliance) |
| Document Type | Assembly Drawing / BOM / CAD |
| Date Collected | 2024-06-01 |
| Collected By | Team Member — Mechanical Lead |
| File Name (Assembly) | Mech_Custom_DeltaRobot_AssemblyDrawing.pdf |
| File Name (CAD) | Mech_Custom_DeltaRobot_Assembly.step |

# 2. Geometry Parameters

| Parameter | Value | Unit | Description |
| --- | --- | --- | --- |
| Base Platform Radius (Rb) | 150 | mm | Distance from centre to motor axis |
| Moving Platform Radius (Re) | 40 | mm | Distance from centre to ball joint |
| Upper Arm Length (L1) | 200 | mm | Motor shaft to lower rod ball joint |
| Lower Rod Length (L2) | 380 | mm | Centre-to-centre rod length |
| Max Workspace Radius | ±160 | mm | XY plane at Z = −250 mm |
| Max Z Travel | 250 | mm | Vertical stroke |
| Home Position (Z) | −250 | mm | From base plate datum |
| Max Payload (dynamic) | 2 | kg | Including end effector |

# 3. Bill of Materials — Mechanical

| Item | Qty | Description | Material | Standard |
| --- | --- | --- | --- | --- |
| Base plate | 1 | Circular motor mount plate, Ø400 mm | Al 6061-T6 | Custom CNC |
| Motor bracket | 3 | Motor flange-to-base adapter | Al 6061-T6 | Custom CNC |
| Upper arm | 3 | 50×25×2 mm rectangular tube | Al 6061-T6 | Custom CNC |
| Lower rod pair | 6 | Ø8 mm CF tube, L=380 mm | CFRP | Custom |
| Ball joint (M6) | 12 | IGUS KBRM-02 rod end | IGUS iglide J | Standard |
| Moving platform | 1 | Ø100 mm disc, 5 mm thick | Al 5052 | Custom CNC |
| Column / upright | 3 | Ø25 mm round tube, H=300 mm | Al 6061-T6 | Extrusion |
| Frame fasteners | ≥60 | M4, M5, M6 socket head bolts | A2 Stainless | ISO 4762 |

# 4. Surface Treatment & Tolerances

## 4.1 Machined Parts

- All aluminium parts: anodise Type II, 15 µm, clear (or black for aesthetics).

- Critical bores (motor fit, bearing seats): H7 tolerance (±0.015 mm).

- General machined features: ±0.1 mm unless noted on drawing.

## 4.2 Carbon Fibre Rods

- Rod straightness: < 0.3 mm over 380 mm length.

- Adhesive bond (rod to ball joint insert): Loctite Hysol EA-9394 structural epoxy.

- Cure: 24 h at room temperature or 1 h at 65 °C.

## 4.3 Ball Joints

- Radial play: < 0.02 mm (IGUS KBRM-02 dry-running).

- Tightening torque (M6): 8 N·m with medium-strength thread-locker (Loctite 243).

# 5. Required Documents Checklist

| Document | Priority | Status | File Name |
| --- | --- | --- | --- |
| Assembly Drawing (2D) | Required | Collect | Mech_Custom_DeltaRobot_AssemblyDwg.pdf |
| Full BOM (Excel) | Required | Collect | Mech_Custom_DeltaRobot_BOM.xlsx |
| STEP Assembly File | Required | Collect | Mech_Custom_DeltaRobot_Assembly.step |
| Part Drawings (each CNC part) | Required | Collect | Mech_Custom_[PartName]_Dwg.pdf |
| FEA / Stress Analysis Report | Preferred | Collect | Mech_Custom_DeltaRobot_FEA.pdf |
| Kinematics Analysis (workspace) | Preferred | Collect | Mech_Custom_DeltaRobot_Kinematics.pdf |
| Exploded Assembly View | Preferred | Collect | Mech_Custom_DeltaRobot_Exploded.pdf |
| DFM / DFA Notes | Advanced | Collect | Mech_Custom_DeltaRobot_DFM.pdf |